BHARAT ELECTRONICS LIMITED (BEL) — PROBATIONARY ENGINEER (ELECTRONICS) 200 High-Yield GATE Electronics One-Liner Questions, Solving Hints & Complete Answer Key Curated from GATE ECE 1-Mark PYQs & PSU Question Banks aligned with BEL May 2025 & Dec 2025 Exam Patterns • 8 Core Subjects (25 Qs/Subject)

Electronic Devices & VLSI

Q.1 In an n-type silicon sample at thermal equilibrium, if the donor concentration is ND = 1016 cm−3 and the intrinsic carrier concentration is ni = 1.5 × 1010 cm−3, what is the minority hole concentration p0? (A) 1.5 × 104 cm−3 (B) 2.25 × 104 cm−3 (C) 1.0 × 106 cm−3 (D) 2.25 × 1010 cm−3

Q.2 According to the Einstein relation in a non-degenerate semiconductor at temperature T = 300 K (where thermal voltage VT ≈ 25.9 mV), if the electron mobility is μn = 1000 cm2/(V⋅s), the electron diffusion coefficient Dn is approximately: (A) 12.95 cm2/s (B) 25.9 cm2/s (C) 38.6 cm2/s (D) 51.8 cm2/s

Q.3 When the doping concentration on both sides of an abrupt p-n junction is increased, the width of the depletion region W and the breakdown voltage VBR change as: (A) Both W and VBR increase (B) W decreases and VBR decreases (C) W decreases and VBR increases (D) W increases and VBR decreases

Q.4 A silicon p-n junction diode with reverse Breakdown Voltage VZ < 4 V breaks down primarily due to: (A) Avalanche multiplication (B) Zener (quantum mechanical tunneling) breakdown (C) Thermal runaway (D) Punch-through effect

Q.5 What is the temperature coefficient of breakdown voltage for Zener breakdown and Avalanche breakdown, respectively? (A) Positive, Negative (B) Negative, Positive (C) Positive, Positive (D) Negative, Negative

Q.6 The diffusion capacitance CD of a forward-biased p-n junction diode varies with the forward DC diode current ID and minority carrier lifetime τ as: (A) Directly proportional to both ID and τ (B) Inversely proportional to ID and directly proportional to τ (C) Directly proportional to √ID and independent of τ (D) Inversely proportional to both ID and τ

Q.7 For an abrupt p+-n junction under reverse bias VR (where VR ≫ Vbi), the junction transition capacitance Cj is proportional to: (A) VR−1/2 (B) VR−1/3 (C) VR1/2 (D) VR−1

Q.8 A direct-bandgap semiconductor LED has a bandgap energy Eg = 1.55 eV. What is the approximate wavelength λ of the emitted light? (Take hc ≈ 1.24 eV⋅μm) (A) 0.62 μm (620 nm) (B) 0.80 μm (800 nm) (C) 1.30 μm (1300 nm) (D) 1.55 μm (1550 nm)

Q.9 Which of the following materials is an indirect bandgap semiconductor and therefore unsuitable for high-efficiency light-emitting diodes (LEDs)? (A) GaAs (B) InP (C) Crystalline Silicon (Si) (D) GaN

Q.10 In a PIN photodiode, the intrinsic (i) layer between the p+ and n+ regions is introduced primarily to: (A) Increase junction capacitance and reduce quantum efficiency (B) Reduce junction capacitance and increase the depletion width for higher photon absorption (C) Provide internal avalanche multiplication gain (D) Allow operation in forward bias

Q.11 In a p-substrate MOS capacitor, when a large positive gate voltage (VG > VT) is applied, the High-Frequency (HF) C-V curve differs from the Low-Frequency (LF) C-V curve in strong inversion because: (A) Majority holes cannot follow the high-frequency AC signal (B) Minority electrons in the inversion layer cannot recombine/generate fast enough to follow the high-frequency AC signal (C) Oxide capacitance disappears at high frequencies (D) Depletion width shrinks to zero at high frequencies

Q.12 For an ideal MOS capacitor with gate oxide thickness tox = 10 nm and relative permittivity of SiO2 equal to εox = 3.9 (with ε0 = 8.854 × 10−14 F/cm), the oxide capacitance per unit area Cox is approximately: (A) 34.5 nF/cm2 (B) 345 nF/cm2 (C) 3.45 μF/cm2 (D) 172.5 nF/cm2

Q.13 The condition for the onset of strong inversion in a p-substrate MOS capacitor (with bulk Fermi potential φF) is that the surface potential φs equals: (A) φF (B) 2φF (C) −φF (D) 0 V

Q.14 An n-channel enhancement MOSFET with threshold voltage VTH = 1 V is biased with VGS = 3 V and VDS = 1.5 V. In which region is the MOSFET operating? (A) Cut-off region (B) Linear (Triode) region (C) Saturation region (D) Subthreshold region

Q.15 In the saturation region of an n-channel MOSFET (neglecting channel-length modulation), the drain current ID and transconductance gm are related to the overdrive voltage VOV = (VGS − VTH) as: (A) ID ∝ VOV and gm is constant (B) ID ∝ VOV2 and gm = 2ID / VOV (C) ID ∝ VOV2 and gm = ID / (2VOV) (D) ID ∝ eV_{OV} and gm = ID / VT

Q.16 In a MOSFET, the phenomenon where the effective channel length Leff decreases as the drain-to-source voltage VDS increases in saturation is known as: (A) Body effect (B) Channel-length modulation (C) Velocity saturation (D) Drain-Induced Barrier Lowering (DIBL)

Q.17 When a reverse bias VSB > 0 is applied between the source and substrate (body) of an n-channel MOSFET, the threshold voltage VTH: (A) Decreases linearly with VSB (B) Increases due to the body (substrate bias) effect (C) Remains completely unchanged (D) Drops to zero

Q.18 In a bipolar junction transistor (BJT) operating in the forward-active mode, the decrease of effective base width with increasing reverse collector-base voltage VCB is called: (A) Kirk effect (B) Early effect (base-width modulation) (C) Gummel effect (D) Webster effect

Q.19 If a BJT has a common-base DC current gain α = 0.98, what are its common-emitter DC current gain β and common-collector current gain γ = IE/IB, respectively? (A) β = 49, γ = 50 (B) β = 50, γ = 49 (C) β = 98, γ = 99 (D) β = 48, γ = 49

Q.20 In a BJT, when both the emitter-base junction and the collector-base junction are reverse-biased, the transistor operates in the: (A) Forward-active region (B) Reverse-active region (C) Saturation region (D) Cut-off region

Q.21 During the Deal-Grove thermal oxidation of silicon in IC fabrication, dry O2 oxidation compared to wet (H2O steam) oxidation produces: (A) Faster growth rate and lower dielectric breakdown strength (B) Slower growth rate, denser oxide, and higher quality Si-SiO2 interface (used for thin gate oxides) (C) Thick field oxides in much shorter time (D) Porous oxide layers unsuitable for MOSFET gates

Q.22 In CMOS IC fabrication, why is ion implantation generally preferred over high-temperature thermal diffusion for doping source, drain, and threshold-adjust regions? (A) Ion implantation requires no annealing step afterward (B) Ion implantation allows precise control of dopant dose and depth profile at lower temperatures (C) Ion implantation causes zero crystal lattice damage (D) Ion implantation is isotropic and spreads dopants wide under the gate

Q.23 In the Hall effect experiment on an extrinsic semiconductor, if the measured Hall coefficient RH is negative, the semiconductor is: (A) Intrinsic (B) p-type (C) n-type (D) Compensated with equal electron and hole mobility

Q.24 A solar cell has an open-circuit voltage Voc = 0.6 V, a short-circuit current Isc = 2 A, and a fill factor FF = 0.75. The maximum electrical power Pmax delivered by the solar cell is: (A) 0.80 W (B) 0.90 W (C) 1.20 W (D) 1.60 W

Q.25 In a Schottky barrier diode (metal-semiconductor rectifying contact), current conduction is primarily due to: (A) Minority carrier injection and storage, resulting in slow reverse recovery (B) Majority carrier thermionic emission over the barrier, resulting in near-zero reverse recovery time (C) Avalanche multiplication at the metal interface (D) Hole diffusion in the metal layer

Analog Electronics

Q.26 In a half-wave rectifier circuit with a peak sinusoidal secondary voltage Vm and no filter capacitor, the Peak Inverse Voltage (PIV) across the diode and the ripple factor are: (A) PIV = Vm, Ripple factor = 1.21 (B) PIV = 2Vm, Ripple factor = 0.482 (C) PIV = Vm, Ripple factor = 0.482 (D) PIV = 2Vm, Ripple factor = 1.21

Q.27 A center-tapped full-wave rectifier is fed with a transformer secondary voltage of 10 sin(ωt) V on each half-winding. Assuming ideal diodes, the Peak Inverse Voltage (PIV) across each diode is: (A) 5 V (B) 10 V (C) 20 V (D) 10√2 V

Q.28 A positive clamper circuit (with an ideal diode and a capacitor) is fed with a 10 V peak-to-peak square wave (±5 V symmetric about 0 V). Under steady-state conditions, the output voltage waveform swings between: (A) −10 V and 0 V (B) 0 V and +10 V (C) −5 V and +5 V (D) +5 V and +15 V

Q.29 In a Zener diode voltage regulator, the Zener diode has VZ = 5 V and minimum knee current IZK = 2 mA. If the series resistor is RS = 100 Ω and the DC input is Vin = 10 V, what is the maximum load current IL(max) that can be drawn while maintaining regulation? (A) 50 mA (B) 48 mA (C) 52 mA (D) 25 mA

Q.30 Among the standard BJT biasing configurations, which circuit provides the best stabilization of the quiescent operating point (Q-point) against variations in transistor current gain β? (A) Fixed-bias circuit (B) Collector-to-base bias circuit (C) Voltage-divider bias with emitter resistor RE (D) Emitter-follower without base resistors

Q.31 A BJT is biased at a quiescent collector current ICQ = 2.6 mA at room temperature (VT = 26 mV) and has β = 100. Its small-signal transconductance gm and input resistance rπ (or hie) are: (A) gm = 100 mS, rπ = 1 kΩ (B) gm = 50 mS, rπ = 2 kΩ (C) gm = 10 mS, rπ = 10 kΩ (D) gm = 100 mS, rπ = 10 kΩ

Q.32 In a Common-Emitter (CE) amplifier with an unbypassed emitter resistor RE, introducing RE into the emitter leg causes: (A) Input resistance to decrease and voltage gain to increase (B) Input resistance to increase to rπ + (1 + β)RE and voltage gain magnitude to decrease (improving linearity and bandwidth) (C) Output resistance to drop to zero (D) Phase shift between input and output to become 0°

Q.33 Which single-stage BJT amplifier configuration provides a voltage gain close to unity (Av ≈ 1), high input impedance, low output impedance, and 0° phase shift? (A) Common-Emitter (CE) (B) Common-Base (CB) (C) Common-Collector (CC / Emitter Follower) (D) Cascode (CE–CB)

Q.34 Why is a Cascode amplifier (Common-Emitter stage driving a Common-Base stage) widely used in high-frequency analog design? (A) It eliminates coupling capacitors at low frequencies (B) It minimizes the Miller capacitance effect on the input stage, significantly extending the upper 3-dB cutoff frequency (C) It provides unity current gain and low output impedance (D) It operates without any DC bias supply

Q.35 In an inverting amplifier with voltage gain Av = −99, a feedback capacitor Cf = 2 pF is connected between the input and output terminals. By Miller's theorem, the equivalent input Miller capacitance CM(in) seen at the input node is: (A) 2 pF (B) 196 pF (C) 198 pF (D) 200 pF

Q.36 In a differential amplifier, if the differential-mode gain is Ad = 2000 and the common-mode gain is Acm = 0.02, what is the Common-Mode Rejection Ratio (CMRR) in decibels (dB)? (A) 50 dB (B) 80 dB (C) 100 dB (D) 120 dB

Q.37 For a Class-B push-pull power amplifier driving a resistive load with a sinusoidal signal, the maximum theoretical conversion efficiency ηmax is: (A) 25% (B) 50% (C) 78.5% (π/4) (D) 90%

Q.38 Crossover distortion in a Class-B push-pull power amplifier occurs because of the base-emitter cut-in voltage (~0.7 V) of the transistors and is eliminated by: (A) Operating the amplifier in Class-C mode (B) Biasing the transistors slightly into conduction using a Class-AB diode-compensated biasing network (C) Increasing the load resistance (D) Adding a low-pass filter at the input

Q.39 Negative Voltage-Series feedback (such as in a non-inverting Op-Amp amplifier) modifies the input impedance Rin and output impedance Rout of the amplifier by: (A) Increasing Rin and decreasing Rout (B) Decreasing Rin and increasing Rout (C) Increasing both Rin and Rout (D) Decreasing both Rin and Rout

Q.40 An amplifier has an open-loop midband gain A = 1000 and an upper 3-dB cutoff frequency fH = 10 kHz. If negative feedback with feedback factor β = 0.009 is applied, the closed-loop gain Af and new upper cutoff frequency fHf are: (A) Af = 100, fHf = 100 kHz (B) Af = 100, fHf = 1 kHz (C) Af = 10, fHf = 1 MHz (D) Af = 90, fHf = 90 kHz

Q.41 In an ideal Op-Amp non-inverting amplifier with input resistor R1 = 2 kΩ (connected from inverting terminal to ground) and feedback resistor Rf = 18 kΩ, if Vin = 0.5 V is applied to the non-inverting terminal, the output voltage Vout is: (A) −4.5 V (B) +4.5 V (C) +5.0 V (D) +5.5 V

Q.42 An Op-Amp has a Slew Rate SR = 2 V/μs. What is the maximum peak amplitude Vm of a 100 kHz sinusoidal output signal that the Op-Amp can reproduce without slew-rate distortion? (A) (10 / π) V ≈ 3.18 V (B) (20 / π) V ≈ 6.37 V (C) 20 V (D) 2π V ≈ 6.28 V

Q.43 In an Op-Amp Schmitt Trigger (regenerative comparator), positive feedback is applied to the non-inverting terminal primarily to: (A) Ensure linear amplification with low harmonic distortion (B) Introduce hysteresis (two distinct threshold voltages VUTP and VLTP) for high noise immunity (C) Integrate the input waveform (D) Reduce the slew rate of the Op-Amp

Q.44 According to the Barkhausen criterion for sustained sinusoidal oscillations in a feedback oscillator with loop gain A(jω)β(jω), at the oscillation frequency ω0: (A) ¦A(jω0)β(jω0)¦ = 1 and ∠A(jω0)β(jω0) = 180° (B) ¦A(jω0)β(jω0)¦ = 1 and ∠A(jω0)β(jω0) = 0° (or 360°) (C) ¦A(jω0)β(jω0)¦ < 1 and ∠A(jω0)β(jω0) = 0° (D) ¦A(jω0)β(jω0)¦ = 0 and ∠A(jω0)β(jω0) = 90°

Q.45 In an RC Phase-Shift oscillator using three identical RC sections and an inverting amplifier, the oscillation frequency f0 and minimum required amplifier gain magnitude ¦A¦ are: (A) f0 = 1 / (2π√6 RC), ¦A¦ ≥ 29 (B) f0 = 1 / (2π RC), ¦A¦ ≥ 3 (C) f0 = √6 / (2π RC), ¦A¦ ≥ 29 (D) f0 = 1 / (2π√3 RC), ¦A¦ ≥ 8

Q.46 In a Wien-Bridge oscillator using an Op-Amp non-inverting amplifier with resistors R1 and Rf, and balanced bridge arms R and C, the frequency of oscillation f0 and required non-inverting gain (1 + Rf/R1) are: (A) f0 = 1 / (2π RC), Gain = 3 (so Rf = 2R1) (B) f0 = 1 / (2π√6 RC), Gain = 29 (C) f0 = 1 / (2π RC), Gain = 1 (D) f0 = 1 / (4π RC), Gain = 2

Q.47 A Colpitts oscillator has a tank circuit consisting of an inductor L = 100 μH and two series capacitors C1 = 200 pF and C2 = 200 pF. Its resonant oscillation frequency f0 is: (A) 1 / (2π × 10−7) Hz ≈ 1.59 MHz (B) 1 / (4π × 10−7) Hz ≈ 796 kHz (C) 1 / (π × 10−7) Hz ≈ 3.18 MHz (D) 5.0 MHz

Q.48 In a 555 Timer IC internal block diagram, the three internal voltage-divider resistors (each of value 5 kΩ) establish upper and lower comparator reference voltages equal to: (A) (1/2) VCC and (1/4) VCC (B) (2/3) VCC and (1/3) VCC (C) (3/4) VCC and (1/4) VCC (D) (4/5) VCC and (1/5) VCC

Q.49 In a 555 Timer configured as an astable multivibrator with resistors RA, RB and timing capacitor C, the capacitor voltage vC(t) charges and discharges periodically between: (A) 0 V and VCC (B) (1/3) VCC and (2/3) VCC (C) 0 V and (2/3) VCC (D) (1/2) VCC and VCC

Q.50 Among standard second-order active filter approximations, which filter provides a maximally flat magnitude response (zero ripple) in the passband? (A) Chebyshev Type-I filter (B) Butterworth filter (C) Elliptic (Cauer) filter (D) Bessel-Thomson filter

Network Theory

Q.51 A connected planar electrical network has N = 6 nodes and B = 10 branches. What are the minimum number of nodal equations and independent loop (mesh) equations required to solve the network? (A) 5 nodal, 5 mesh (B) 6 nodal, 4 mesh (C) 5 nodal, 6 mesh (D) 4 nodal, 6 mesh

Q.52 Three equal resistors of value R = 9 Ω each are connected in a Delta (Δ) configuration. When converted to an equivalent Star (Y) configuration, the resistance of each branch in the Star network is: (A) 27 Ω (B) 9 Ω (C) 3 Ω (D) 4.5 Ω

Q.53 While finding the Thevenin resistance RTh of a linear circuit containing both independent sources and dependent sources, how should the sources be treated? (A) Deactivate both independent and dependent sources (B) Deactivate only independent sources (short voltage sources, open current sources) and keep dependent sources active, applying a test source at the port (C) Deactivate dependent sources and keep independent sources active (D) Replace all sources with 1 Ω resistors

Q.54 A linear AC source has an internal Thevenin impedance ZTh = (6 + j8) Ω. For maximum average power transfer to a complex load impedance ZL = RL + jXL, the required load impedance ZL is: (A) (6 + j8) Ω (B) (6 − j8) Ω (C) 10 Ω (purely resistive) (D) (8 − j6) Ω

Q.55 In the same AC source with internal impedance ZTh = (6 + j8) Ω, if the load is restricted to be purely resistive (RL only, with XL = 0), what value of RL maximizes power transfer to the load? (A) 6 Ω (B) 8 Ω (C) 10 Ω (D) 14 Ω

Q.56 For a linear, bilateral two-port network containing no dependent sources, the reciprocity condition in terms of Z-parameters, Y-parameters, and ABCD (transmission) parameters is: (A) Z12 = Z21, Y12 = Y21, and AD − BC = 1 (B) Z11 = Z22, Y11 = Y22, and A = D (C) Z12 = −Z21 and AD − BC = 0 (D) h12 = h21 and A = D

Q.57 When two two-port networks Na and Nb are connected in cascade (output of Na feeding input of Nb), the overall two-port matrix is obtained by multiplying their individual: (A) Z-parameter matrices (B) Y-parameter matrices (C) ABCD (Transmission) parameter matrices (D) h-parameter matrices

Q.58 A T-network has series arms Ra = 2 Ω and Rb = 3 Ω, and a shunt central arm Rc = 5 Ω. Its open-circuit impedance parameters [Z11, Z12; Z21, Z22] are: (A) [7, 5; 5, 8] Ω (B) [2, 5; 5, 3] Ω (C) [7, −5; −5, 8] Ω (D) [5, 2; 3, 5] Ω

Q.59 An uncharged capacitor C = 2 μF is connected in series with a resistor R = 500 kΩ across a 10 V DC voltage source at t = 0 through a switch. What are the initial current i(0+) and the capacitor voltage at t = 1 s? (A) i(0+) = 20 μA, vC(1) ≈ 6.32 V (B) i(0+) = 0 A, vC(1) = 10 V (C) i(0+) = 20 μA, vC(1) ≈ 3.68 V (D) i(0+) = 10 μA, vC(1) ≈ 5.0 V

Q.60 An inductor L = 2 H carrying zero initial current is connected in series with a resistor R = 4 Ω across a 12 V DC step source at t = 0. What are the inductor voltage vL(0+) and steady-state current i(∞)? (A) vL(0+) = 0 V, i(∞) = 3 A (B) vL(0+) = 12 V, i(∞) = 3 A (C) vL(0+) = 12 V, i(∞) = 0 A (D) vL(0+) = 6 V, i(∞) = 6 A

Q.61 In the s-domain (Laplace transform), an inductor L with initial current I0 is represented by a series combination of an impedance sL and a voltage source of value: (A) L I0 (opposing the current) (B) L I0 (aiding the initial current direction) (C) I0 / (sL) (D) s L I0

Q.62 In a series RLC circuit with R = 10 Ω, L = 10 mH, and C = 1 μF, what are the resonant angular frequency ω0, quality factor Q, and half-power bandwidth BW (in rad/s)? (A) ω0 = 104 rad/s, Q = 10, BW = 1000 rad/s (B) ω0 = 104 rad/s, Q = 100, BW = 100 rad/s (C) ω0 = 105 rad/s, Q = 10, BW = 104 rad/s (D) ω0 = 103 rad/s, Q = 1, BW = 1000 rad/s

Q.63 At resonance in a series RLC circuit with quality factor Q = 25 excited by a 10 V RMS source, the RMS voltage across the capacitor VC is: (A) 10 V (B) 250 V (C) 0.4 V (D) 0 V

Q.64 In a parallel RLC circuit excited by a sinusoidal current source at frequency ω > ω0 (above its resonant frequency), the net admittance is dominated by which element and what is the phase relationship? (A) Inductive; current lags voltage (B) Capacitive; current leads voltage (C) Resistive; current is in phase with voltage (D) Capacitive; voltage leads current

Q.65 Two inductors L1 = 4 H and L2 = 9 H have a mutual inductance M = 3 H. When connected in series-opposing configuration, the total equivalent inductance Leq is: (A) 19 H (B) 7 H (C) 10 H (D) 16 H

Q.66 Twelve identical resistors, each of resistance R, are connected to form the 12 edges of a cube. The equivalent resistance Req measured across two diagonally opposite vertices (body diagonal) of the cube is: (A) (5/6) R (B) (3/4) R (C) (7/12) R (D) (1/2) R

Q.67 According to Tellegen's theorem, for any lumped electrical network with B branches having branch voltages vk and branch currents ik that satisfy KVL and KCL respectively, the sum Σk=1B vk ik is: (A) Equal to zero only for linear resistive networks (B) Identically zero for any lumped network (linear, nonlinear, time-variant, or passive/active) obeying KVL and KCL (C) Equal to the reactive power stored in inductors (D) Always positive

Q.68 The driving-point impedance of a network is given by Z(s) = (s + 3) / (s + 1). Since the pole is at s = −1 and the zero is at s = −3 (alternating along the negative real axis with pole closest to the origin), this network can be realized as a: (A) Foster / Cauer R–C driving-point impedance (B) Foster / Cauer R–L driving-point impedance (C) Purely L–C lossless reactance network (D) Non-minimum phase unstable network

Q.69 A periodic current waveform has a DC component of 3 A and a fundamental sinusoidal component of peak amplitude 4√2 A. What is the true RMS value of the current? (A) 5 A (B) 7 A (C) √25 = 5√2 A (D) √41 A

Q.70 An AC load draws 4 kW of active power (P) and 3 kVAR of inductive reactive power (Q) from a 250 V RMS supply. What are the apparent power S and the power factor cos(φ) of the load? (A) S = 5 kVA, cos(φ) = 0.8 lagging (B) S = 7 kVA, cos(φ) = 0.75 lagging (C) S = 5 kVA, cos(φ) = 0.8 leading (D) S = 5 kVA, cos(φ) = 0.6 lagging

Q.71 In a balanced 3-phase Star (Y)-connected system with line-to-line voltage VL and line current IL, the phase voltage Vph and phase current Iph are: (A) Vph = VL / √3 and Iph = IL (B) Vph = VL and Iph = IL / √3 (C) Vph = √3 VL and Iph = IL (D) Vph = VL / 3 and Iph = 3 IL

Q.72 A constant-resistance symmetrical lattice (or bridged-T) network has series arm impedance Z1 = jωL and shunt/cross arm impedance Z2 = 1/(jωC). The network behaves as an all-pass / constant-resistance network of characteristic resistance R0 at all frequencies if: (A) Z1 + Z2 = R0 (B) Z1 Z2 = L/C = R02 (C) Z1 / Z2 = R02 (D) ω2 L C = 1

Q.73 The Laplace transform of the unit-ramp signal r(t) = t u(t) and the unit-parabolic signal p(t) = (t2/2) u(t) are, respectively: (A) 1/s2 and 1/s3 (B) 1/s and 1/s2 (C) 1/s2 and 2/s3 (D) s and s2

Q.74 If I(s) = (4s + 10) / [s(s + 2)] is the Laplace transform of a current i(t), then by the Initial Value Theorem and Final Value Theorem, i(0+) and i(∞) are: (A) i(0+) = 4, i(∞) = 5 (B) i(0+) = 5, i(∞) = 4 (C) i(0+) = 0, i(∞) = 5 (D) i(0+) = 4, i(∞) = 10

Q.75 In graph theory of electrical networks, if B is the number of branches and N is the number of nodes in a connected graph, the number of twigs (tree branches) and links (chords) in any tree are: (A) Twigs = N − 1, Links = B − N + 1 (B) Twigs = N, Links = B − N (C) Twigs = B − N + 1, Links = N − 1 (D) Twigs = B − 1, Links = N

Signals & Systems and DSP

Q.76 What is the total energy E of the continuous-time signal x(t) = 2 e−4t u(t)? (A) 0.25 J (B) 0.50 J (C) 1.00 J (D) 2.00 J

Q.77 What is the average power P of the continuous-time signal x(t) = 4 + 6 cos(10t + 30°) + 8 sin(20t)? (A) 66 W (B) 116 W (C) 50 W (D) 34 W

Q.78 Consider the continuous-time system described by y(t) = t ⋅ x(t − 2). This system is: (A) Linear and Time-Invariant (LTI) (B) Linear and Time-Varying (LTV) (C) Non-linear and Time-Invariant (D) Non-linear and Non-causal

Q.79 Two continuous-time rectangular pulses x1(t) of duration T1 = 3 s and x2(t) of duration T2 = 5 s are convolved: y(t) = x1(t) * x2(t). What is the shape and total duration of y(t)? (A) Rectangular pulse of duration 8 s (B) Trapezoidal pulse of total duration 8 s (C) Triangular pulse of duration 5 s (D) Trapezoidal pulse of duration 15 s

Q.80 What is the value of the integral I = ∫−∞∞ (t2 + 3t + 5) δ(t − 2) dt? (A) 5 (B) 15 (C) 0 (D) 11

Q.81 A discrete-time LTI system has impulse response h[n] = (0.5)n u[n]. What is its BIBO stability and its DC gain H(ej0)? (A) BIBO stable, DC gain = 2 (B) BIBO stable, DC gain = 0.5 (C) Unstable, DC gain = ∞ (D) BIBO stable, DC gain = 1

Q.82 If a continuous-time periodic signal x(t) is both real and even (x(t) = x(−t)), then its exponential Fourier series coefficients ck are: (A) Purely imaginary and odd (B) Real and even (ck = c−k) (C) Complex conjugates with non-zero phase (D) Zero for all even harmonics

Q.83 According to Parseval's theorem, what is the total energy E of the signal x(t) = 4 sinc(2t), where sinc(u) = sin(πu)/(πu)? (A) 4 J (B) 8 J (C) 16 J (D) 2 J

Q.84 If x(t) has continuous-time Fourier transform X(f), then by the duality and scaling properties, the Fourier transform of x(a t) (for a ≠ 0) is: (A) (1 / ¦a¦) X(f / a) (B) ¦a¦ X(a f) (C) X(f − a) (D) (1 / a) X(a f)

Q.85 What is the unilateral Laplace transform of the signal x(t) = e−2t cos(3t) u(t)? (A) 3 / [(s + 2)2 + 9] (B) (s + 2) / [(s + 2)2 + 9] (C) (s − 2) / [(s − 2)2 + 9] (D) s / [(s + 2)2 + 3]

Q.86 A continuous-time LTI system has transfer function H(s) = (s − 2) / [(s + 1)(s − 3)]. If the system is known to be BIBO stable, what are its Region of Convergence (ROC) and causality? (A) ROC: Re(s) > 3; Causal (B) ROC: −1 < Re(s) < 3; Two-sided (Non-causal) (C) ROC: Re(s) < −1; Anti-causal (D) ROC: Re(s) > −1; Causal

Q.87 The signal x(t) = cos(200πt) + 3 sin(500πt) is multiplied by x2(t) = cos(300πt) to form y(t) = x(t) ⋅ x2(t). What is the Nyquist sampling rate fN for the product signal y(t)? (A) 500 Hz (B) 800 Hz (C) 400 Hz (D) 1000 Hz

Q.88 A pure sinusoidal tone of frequency f0 = 1.2 kHz is sampled at fs = 1.0 kHz and passed through an ideal low-pass reconstruction filter with cutoff frequency fc = fs/2 = 500 Hz. What is the frequency of the reconstructed output sinusoid? (A) 1.2 kHz (B) 200 Hz (C) 800 Hz (D) 400 Hz

Q.89 What is the Z-transform and Region of Convergence (ROC) of the left-sided sequence x[n] = −an u[−n − 1]? (A) X(z) = 1 / (1 − a z−1), ROC: ¦z¦ > ¦a¦ (B) X(z) = 1 / (1 − a z−1), ROC: ¦z¦ < ¦a¦ (C) X(z) = −1 / (1 − a z−1), ROC: ¦z¦ < ¦a¦ (D) X(z) = 1 / (1 + a z−1), ROC: ¦z¦ > ¦a¦

Q.90 If x[n] ↔ X(z) with ROC R, then by the z-domain differentiation property, the Z-transform of n ⋅ x[n] is: (A) −z dX(z)/dz (B) z dX(z)/dz (C) −dX(z)/dz (D) X(z) / z

Q.91 Given two finite-length sequences x1[n] = {1, 2, 3} (of length N1 = 3) and x2[n] = {1, 1, 1, 1} (of length N2 = 4), what is the minimum N-point circular convolution length N required so that circular convolution equals linear convolution? (A) N = 4 (B) N = 6 (C) N = 7 (D) N = 12

Q.92 For an 8-point Discrete Fourier Transform (DFT) of a real sequence x[n], if X[2] = 3 − j4, then by conjugate symmetry X[6] is: (A) 3 − j4 (B) 3 + j4 (C) −3 + j4 (D) −3 − j4

Q.93 In a Radix-2 Cooley-Tukey Fast Fourier Transform (FFT) algorithm for N = 1024 points, how many complex multiplications are required compared to direct DFT computation? (A) 5,120 (vs. 1,048,576 for direct DFT) (B) 10,240 (vs. 1,048,576 for direct DFT) (C) 1,024 (vs. 10,240 for direct DFT) (D) 2,048 (vs. 512,000 for direct DFT)

Q.94 A causal FIR filter of length M = 5 has a symmetric impulse response h[n] = {2, −1, 4, −1, 2} for n = 0, 1, 2, 3, 4. What is the constant group delay τg (in samples) of this linear-phase FIR filter? (A) 5 samples (B) 2.5 samples (C) 2 samples (D) 4 samples

Q.95 When converting a stable continuous-time analog filter Ha(s) into a discrete-time IIR filter H(z) using the Bilinear Transform s = (2/T)(z − 1)/(z + 1), the entire left-half of the s-plane (Re(s) < 0) maps to: (A) The exterior of the unit circle (¦z¦ > 1) (B) The interior of the unit circle (¦z¦ < 1), guaranteeing IIR stability and avoiding aliasing (C) The real axis of the z-plane (D) The right half of the z-plane

Q.96 In FIR filter design using window functions, why is a Hamming or Kaiser window preferred over a Rectangular window of the same length? (A) It gives a narrower main-lobe width (sharper transition band) than the Rectangular window (B) It significantly reduces the peak side-lobe level (suppressing Gibbs phenomenon ripples in the stopband) at the cost of a wider main lobe (C) It converts the FIR filter into an IIR filter (D) It requires zero multiplications

Q.97 A causal discrete-time LTI system has transfer function H(z) = (z−1 − 0.5) / (1 − 0.5 z−1). Since its pole is at z = 0.5 and its zero is at z = 2 = 1/0.5*, this system is a: (A) Minimum-phase system (B) Stable All-Pass system with ¦H(ejω)¦ = 1 for all ω (C) Linear-phase FIR filter (D) Low-pass notch filter

Q.98 For a causal and stable LTI system to be a Minimum-Phase system, where must all of its poles and zeros be located? (A) Poles inside the unit circle (or left-half s-plane), zeros anywhere (B) Both poles and zeros strictly inside the unit circle (for discrete time) or strictly in the open left-half s-plane (for continuous time) (C) Poles and zeros on the unit circle (D) Zeros in the right-half s-plane

Q.99 What is the fundamental period N0 of the discrete-time sinusoidal signal x[n] = cos(3πn / 7)? (A) 7 (B) 14 (C) 3 (D) 21

Q.100 If a discrete-time signal x[n] is downsampled (decimated) by an integer factor M = 3 without an anti-aliasing low-pass filter, the spectrum Xd(ejω) of the downsampled signal y[n] = x[3n] experiences: (A) Frequency compression by a factor of 3 (B) Frequency stretching (expansion) by a factor of 3 and superposition of 3 shifted copies, which can cause aliasing if X(ejω) is not bandlimited to π/3 (C) Zero change in bandwidth (D) Time-domain interpolation

Control Systems

Q.101 In Mason's gain formula T = (1/Δ) Σ Pk Δk for a Signal Flow Graph (SFG), if the graph has three individual feedback loops with loop gains L1, L2, and L3, and loops L1 and L3 do not touch each other (while L2 touches both), the determinant Δ of the graph is: (A) 1 − (L1 + L2 + L3) (B) 1 − (L1 + L2 + L3) + L1 L3 (C) 1 + (L1 + L2 + L3) − L1 L3 (D) 1 − (L1 + L2 + L3) + L1 L2 L3

Q.102 A first-order control system has closed-loop transfer function T(s) = 1 / (5s + 1). What are its time constant τ and its 2% settling time ts (using the standard 4τ criterion)? (A) τ = 5 s, ts = 20 s (B) τ = 0.2 s, ts = 0.8 s (C) τ = 5 s, ts = 15 s (D) τ = 1 s, ts = 5 s

Q.103 A standard second-order system has closed-loop transfer function T(s) = 100 / (s2 + 12s + 100). What are its undamped natural frequency ωn and damping ratio ζ? (A) ωn = 10 rad/s, ζ = 0.6 (B) ωn = 100 rad/s, ζ = 0.6 (C) ωn = 10 rad/s, ζ = 1.2 (D) ωn = 12 rad/s, ζ = 0.5

Q.104 For an underdamped second-order system with undamped natural frequency ωn = 5 rad/s and damping ratio ζ = 0.6, what are the damped oscillation frequency ωd and the peak time tp for a unit-step input? (A) ωd = 4 rad/s, tp = π/4 s (B) ωd = 3 rad/s, tp = π/3 s (C) ωd = 4 rad/s, tp = π/2 s (D) ωd = 5 rad/s, tp = π/5 s

Q.105 What is the percentage peak overshoot (%Mp) of a second-order unity-feedback system when the damping ratio is ζ = 1/√2 ≈ 0.707? (A) 0% (B) 4.32% (C) 16.3% (D) 25.4%

Q.106 A unity negative-feedback system has open-loop transfer function G(s) = 20 / [s(s + 2)(s + 5)]. What are the Type of the system, the velocity error constant Kv, and the steady-state error ess for a unit-ramp input r(t) = t u(t)? (A) Type 1, Kv = 2, ess = 0.5 (B) Type 1, Kv = 20, ess = 0.05 (C) Type 3, Kv = 2, ess = 0 (D) Type 0, Kv = 0, ess = ∞

Q.107 For a Type-2 unity-feedback control system, the steady-state errors for a unit-step input, a unit-ramp input, and a unit-parabolic input (t2/2) u(t) are, respectively: (A) 0, Finite non-zero, ∞ (B) 0, 0, Finite non-zero (1/Ka) (C) Finite non-zero, ∞, ∞ (D) 0, 0, 0

Q.108 By the Routh-Hurwitz criterion, how many roots of the characteristic polynomial P(s) = s3 + 2s2 + 4s + 24 = 0 lie in the open right-half of the s-plane (Re(s) > 0)? (A) 0 (B) 1 (C) 2 (D) 3

Q.109 A unity-feedback system has characteristic equation s3 + 6s2 + 11s + (6 + K) = 0. For what range of gain K is the closed-loop system asymptotically stable? (A) K > 0 (B) −6 < K < 60 (C) 0 < K < 60 (D) K > 60

Q.110 While constructing a Routh array, if an entire row of zeros appears (and the auxiliary polynomial is formed from the preceding row), this indicates the presence of: (A) Only negative real roots (B) Roots located symmetrically about the origin in the s-plane (such as conjugate imaginary roots ±jω) (C) A single pole at s = −∞ (D) Zero steady-state error

Q.111 An open-loop transfer function has P = 4 finite poles at s = 0, −1, −2, −5 and Z = 1 finite zero at s = −3. In the Root Locus plot (for K > 0), what are the angles of the asymptotes and the centroid σA on the real axis? (A) Angles: 60°, 180°, 300°; Centroid σA = −5/3 (B) Angles: 45°, 135°, 225°, 315°; Centroid σA = −2 (C) Angles: 90°, 270°; Centroid σA = −5/2 (D) Angles: 60°, 180°, 300°; Centroid σA = −8/3

Q.112 A unity-feedback system has open-loop transfer function G(s)H(s) = K / [s(s + 4)]. At what point on the real axis do the two root locus branches break away into the complex plane? (A) s = −1 (B) s = −2 (C) s = −4 (D) s = 0

Q.113 In a Bode magnitude plot, a transfer function with a net excess of (P − Z) = 3 poles over zeros at high frequencies has a high-frequency asymptotic magnitude slope of: (A) −20 dB/decade (−6 dB/octave) (B) −40 dB/decade (−12 dB/octave) (C) −60 dB/decade (−18 dB/octave) (D) −80 dB/decade

Q.114 If the low-frequency asymptote of the Bode magnitude plot of a unity-feedback open-loop transfer function G(jω) has a slope of −40 dB/decade, the system is of: (A) Type 0 (B) Type 1 (C) Type 2 (D) Type 3

Q.115 At the gain crossover frequency ωgc (where ¦G(jωgc)H(jωgc)¦ = 1 or 0 dB), if the open-loop phase angle is ∠G(jωgc)H(jωgc) = −135°, what is the Phase Margin (PM) of the system? (A) −45° (B) +45° (C) +135° (D) +315°

Q.116 An open-loop transfer function G(s)H(s) has P = 1 pole in the open right-half s-plane (RHP). According to the Nyquist stability criterion, for the closed-loop system to be stable, the Nyquist contour of G(jω)H(jω) must encircle the critical point (−1 + j0): (A) Zero times (B) Once in the counter-clockwise (CCW) direction (N = −1) (C) Once in the clockwise (CW) direction (D) Twice in the counter-clockwise direction

Q.117 Introducing a pure time delay (dead time) e−sT into the forward path of a feedback control system affects its frequency response by: (A) Decreasing the magnitude ¦G(jω)¦ without changing the phase (B) Adding a phase lag of −ωT radians without changing the magnitude ¦G(jω)¦, thereby reducing the Phase Margin and stability (C) Increasing both Gain Margin and Phase Margin (D) Shifting the gain crossover frequency to infinity

Q.118 A first-order compensator has transfer function Gc(s) = (1 + s τ) / (1 + α s τ) with 0 < α < 1. Because its zero at s = −1/τ is closer to the origin than its pole at s = −1/(ατ), this compensator is a: (A) Phase-Lag compensator (B) Phase-Lead compensator, providing maximum phase lead φm = sin−1((1 − α)/(1 + α)) at ωm = 1/(τ√α) (C) Notch filter (D) Pure PI controller

Q.119 Comparing a Phase-Lead compensator and a Phase-Lag compensator in control system design, which statement is correct? (A) Lead compensator improves steady-state accuracy and slows down transient response (B) Lead compensator increases bandwidth and speeds up transient response (reducing rise/settling time), whereas Lag compensator improves low-frequency gain and steady-state accuracy (C) Lag compensator increases system bandwidth and noise susceptibility (D) Both Lead and Lag compensators decrease Phase Margin

Q.120 In a Proportional-Derivative (PD) controller with transfer function Gc(s) = Kp + Kd s applied to a second-order system, the derivative action primarily: (A) Increases the system Type by 1 and eliminates steady-state ramp error (B) Increases the effective damping ratio ζ, reducing peak overshoot and improving transient stability without changing steady-state step error (C) Reduces high-frequency sensor noise amplification (D) Decreases the natural frequency ωn

Q.121 For a linear time-invariant state-space system dx/dt = A x + B u and y = C x + D u, the transfer function matrix G(s) = Y(s)/U(s) is given by: (A) C (sI − A) B + D (B) C (sI − A)−1 B + D (C) B (sI − A)−1 C + D (D) C (sI + A)−1 B − D

Q.122 An LTI state-space system has system matrix A = [0, 1; −6, −5]. What are the poles (eigenvalues) of the system? (A) s = −2 and s = −3 (B) s = +2 and s = +3 (C) s = −1 and s = −6 (D) s = 0 and s = −5

Q.123 According to Kalman's controllability and observability criteria for an n-dimensional LTI state-space system (A, B, C), the system is completely state controllable if and only if the n × n controllability matrix Qc = [B AB A2B ... An−1B] has: (A) Determinant equal to zero (B) Full rank equal to n (i.e., det(Qc) ≠ 0 for single-input systems) (C) Trace equal to zero (D) All positive entries

Q.124 In a state-space system where the transfer function C(sI − A)−1B exhibits exact pole-zero cancellation, the system is: (A) Both completely controllable and completely observable (B) Either uncontrollable, unobservable, or both (i.e., not minimal) (C) Always unstable (D) Non-linear

Q.125 For the LTI state equation dx/dt = A x, the State Transition Matrix Φ(t) = eAt = L−1{(sI − A)−1} always satisfies which property at t = 0? (A) Φ(0) = 0 (zero matrix) (B) Φ(0) = I (identity matrix) and Φ−1(t) = Φ(−t) (C) Φ(0) = A (D) Φ(t1 + t2) = Φ(t1) + Φ(t2)

Digital Electronics & 8085

Q.126 In the 8-bit 2's complement binary representation, what is the decimal range of signed integers that can be represented, and what is the 8-bit 2's complement representation of −25? (A) −128 to +127; 11100111 (B) −127 to +127; 11100110 (C) −128 to +127; 10011001 (D) 0 to 255; 11100111

Q.127 What is the Gray code equivalent of the 4-bit binary number 11012? (A) 1011 (B) 1010 (C) 1111 (D) 1001

Q.128 According to the Consensus Theorem in Boolean algebra, the expression F = A⋅B + A̅⋅C + B⋅C simplifies to: (A) A⋅B + B⋅C (B) A⋅B + A̅⋅C (C) A̅⋅C + B⋅C (D) A + B + C

Q.129 For n = 4 Boolean variables, how many distinct Boolean functions can be formed in total, and how many of them are self-dual functions? (A) Total = 216 = 65,536; Self-dual = 28 = 256 (B) Total = 16; Self-dual = 8 (C) Total = 24 = 16; Self-dual = 22 = 4 (D) Total = 28 = 256; Self-dual = 24 = 16

Q.130 What is the minimum number of 2-input NAND gates required to implement a 2-input XOR gate Y = A ⊕ B (without complemented inputs available)? (A) 3 (B) 4 (C) 5 (D) 6

Q.131 A 4-variable Boolean function is given by F(A, B, C, D) = Σm(0, 2, 5, 7, 8, 10, 13, 15). After Karnaugh map minimization, the simplified sum-of-products (SOP) expression for F is: (A) B⋅D + B̅⋅D̅ (i.e., B ⊙ D) (B) A⋅C + A̅⋅C̅ (C) B̅⋅D + B⋅D̅ (D) A̅⋅B + C⋅D

Q.132 To construct a 64:1 multiplexer using only 4:1 multiplexers in a hierarchical tree structure, how many 4:1 multiplexers are required in total? (A) 16 (B) 20 (C) 21 (D) 63

Q.133 Why is a Carry-Lookahead Adder (CLA) significantly faster than a Ripple-Carry Adder (RCA) for adding two n-bit binary numbers? (A) CLA uses fewer logic gates than RCA (B) CLA pre-computes carry-generate (Gi = AiBi) and carry-propagate (Pi = Ai ⊕ Bi) signals to produce all carries in parallel, reducing propagation delay from O(n) to O(1) (two gate levels) (C) CLA operates without a clock signal whereas RCA is synchronous (D) CLA uses half-adders instead of full-adders

Q.134 Among the standard digital IC logic families (CMOS, TTL, and ECL), which family has the lowest static power dissipation, and which family has the fastest switching speed (lowest propagation delay)? (A) Lowest static power: CMOS; Fastest speed: ECL (Emitter-Coupled Logic) (B) Lowest static power: ECL; Fastest speed: CMOS (C) Lowest static power: TTL; Fastest speed: CMOS (D) Lowest static power: CMOS; Fastest speed: Standard TTL

Q.135 A CMOS logic gate operates at supply voltage VDD = 2 V, drives an output load capacitance CL = 5 pF, and switches at a clock frequency f = 100 MHz (one full charge-discharge cycle per clock period). Its dynamic power dissipation Pdyn is: (A) 1 mW (B) 2 mW (C) 4 mW (D) 0.5 mW

Q.136 How is the race-around condition (which occurs in a level-triggered JK latch when J = K = 1 and the clock pulse width tp >> propagation delay tpd) eliminated in sequential circuit design? (A) By setting J = 0 and K = 0 permanently (B) By using edge-triggered JK flip-flops or a Master-Slave JK flip-flop topology (C) By increasing the clock pulse width tp (D) By removing the feedback connection from Q to the input gates

Q.137 What is the characteristic equation for the next state Q(t + 1) of a JK flip-flop and a T (Toggle) flip-flop in terms of present state Q? (A) JK: Q(t+1) = J⋅Q̅ + K̅⋅Q; T: Q(t+1) = T ⊕ Q (B) JK: Q(t+1) = J̅⋅Q + K⋅Q̅; T: Q(t+1) = T ⋅ Q (C) JK: Q(t+1) = J + K⋅Q; T: Q(t+1) = T + Q (D) JK: Q(t+1) = J⋅Q + K̅⋅Q̅; T: Q(t+1) = T ⊙ Q

Q.138 A synchronous sequential path between two edge-triggered D flip-flops has flip-flop clock-to-Q propagation delay tcq = 3 ns, combinational logic delay tcomb = 12 ns, and flip-flop setup time tsu = 5 ns (with zero clock skew). What is the maximum allowable clock frequency fmax? (A) 50 MHz (B) 100 MHz (C) 200 MHz (D) 66.7 MHz

Q.139 A 4-stage asynchronous (ripple) binary counter uses 4 flip-flops, each having a propagation delay of tpd = 15 ns. If a Decade (MOD-10) counter is instead built with 4 flip-flops, how many states does it have and what is the worst-case ripple delay of the 4-stage ripple counter? (A) 10 states; worst-case ripple delay = 4 × 15 ns = 60 ns (B) 16 states; worst-case ripple delay = 15 ns (C) 10 states; worst-case ripple delay = 15 ns (D) 8 states; worst-case ripple delay = 45 ns

Q.140 For an n-bit Ring Counter and an n-bit Johnson (Twisted-Ring) Counter, how many valid states are in their primary counting sequences, and what is the output frequency if clocked at fclk = 120 kHz with n = 6? (A) Ring: n = 6 states (20 kHz); Johnson: 2n = 12 states (10 kHz) (B) Ring: 2n = 64 states; Johnson: n = 6 states (C) Ring: 2n = 12 states (10 kHz); Johnson: n = 6 states (20 kHz) (D) Ring: 6 states (60 kHz); Johnson: 12 states (30 kHz)

Q.141 What is the fundamental architectural difference between a Moore Finite State Machine (FSM) and a Mealy Finite State Machine (FSM)? (A) In a Moore FSM, outputs depend only on the present state; in a Mealy FSM, outputs depend on both the present state and the present external inputs (B) In a Moore FSM, outputs change asynchronously with input glitches; in a Mealy FSM, outputs only change on clock edges (C) A Moore FSM always requires fewer states than an equivalent Mealy FSM (D) A Mealy FSM does not use flip-flops

Q.142 An 8-bit Digital-to-Analog Converter (DAC) has a full-scale output voltage of 5.10 V (corresponding to binary input 111111112 = 255). What are its step size (resolution / 1 LSB) and its analog output for input 100000002 (128)? (A) Step size = 20 mV; Output = 2.56 V (B) Step size = 19.92 mV; Output = 2.55 V (C) Step size = 10 mV; Output = 1.28 V (D) Step size = 20 mV; Output = 2.50 V

Q.143 Among the following Analog-to-Digital Converter (ADC) architectures — Flash (Parallel) ADC, Successive Approximation Register (SAR) ADC, and Dual-Slope Integrating ADC — which is the fastest and which has the highest noise rejection? (A) Fastest: Flash ADC (1 clock cycle, requires 2n − 1 comparators); Highest noise rejection: Dual-Slope ADC (B) Fastest: Dual-Slope ADC; Highest noise rejection: Flash ADC (C) Fastest: SAR ADC; Highest noise rejection: Flash ADC (D) Fastest: Flash ADC; Highest noise rejection: Counter-type ADC

Q.144 An 8-bit Successive Approximation Register (SAR) ADC is clocked at fclk = 1 MHz. If each conversion requires n = 8 clock cycles, what is the conversion time Tconv? (A) 1 μs (B) 8 μs (C) 256 μs (D) 0.125 μs

Q.145 To build a 64K × 16 memory system using 16K × 8 RAM chips, how many 16K × 8 RAM chips are required, and how many address lines are needed for the 64K × 16 memory system? (A) 8 chips; 16 address lines (B) 4 chips; 16 address lines (C) 8 chips; 14 address lines (D) 16 chips; 16 address lines

Q.146 Comparing Programmable Logic Array (PLA), Programmable Array Logic (PAL), and PROM for implementing combinational logic functions in sum-of-products form: (A) PROM has fixed AND / programmable OR; PAL has programmable AND / fixed OR; PLA has both programmable AND and programmable OR planes (B) PROM has programmable AND / fixed OR; PAL has fixed AND / programmable OR (C) PLA has fixed AND and fixed OR planes (D) PAL is slower than PLA because both planes are programmable

Q.147 In the Intel 8085 microprocessor, how many address lines and data lines are present, and why is the ALE (Address Latch Enable) signal used? (A) 16 address lines, 8 data lines; ALE demultiplexes the lower-order address/data bus AD0–AD7 (B) 8 address lines, 16 data lines; ALE enables interrupts (C) 20 address lines, 8 data lines; ALE latches the status lines (D) 16 address lines, 16 data lines; ALE resets the program counter

Q.148 Among the hardware interrupts of the Intel 8085 microprocessor (TRAP, RST 7.5, RST 6.5, RST 5.5, and INTR), which interrupt is Non-Maskable, has the highest priority, and what is its vector address? (A) TRAP (RST 4.5); vector address = 0024H (both edge- and level-triggered) (B) RST 7.5; vector address = 003CH (C) INTR; vector address = 0000H (D) TRAP; vector address = 0034H

Q.149 In the 8085 microprocessor, the instruction LHLD 2050H is a 3-byte instruction that copies the contents of memory locations 2050H and 2051H into the H–L register pair. How many machine cycles and T-states does LHLD 2050H require? (A) 5 machine cycles (Opcode Fetch + 2 Memory Reads for address + 2 Memory Reads for data) and 16 T-states (B) 3 machine cycles and 10 T-states (C) 4 machine cycles and 13 T-states (D) 2 machine cycles and 7 T-states

Q.150 What happens to the Stack Pointer (SP) and Program Counter (PC) when the 8085 microprocessor executes a CALL 3000H instruction? (A) The 16-bit return address (next PC) is pushed onto the stack, SP is decremented by 2 (SP ← SP − 2), and PC is loaded with 3000H (B) SP is incremented by 2 (SP ← SP + 2) and PC is loaded with 3000H (C) Only the 8-bit accumulator is pushed onto the stack and SP ← SP − 1 (D) SP remains unchanged

Communication Systems

Q.151 An Amplitude Modulated (AM) transmitter radiates 10 kW of carrier power when unmodulated. When modulated by a single sinusoidal tone with modulation index μ = 0.6, the total transmitted power Pt is: (A) 11.8 kW (B) 13.6 kW (C) 10.6 kW (D) 15.0 kW

Q.152 For a standard single-tone AM wave with 100% modulation (μ = 1), what is the power efficiency η (ratio of total sideband power to total transmitted power) and the fraction of power in a single sideband? (A) Total sideband efficiency = 33.33% (1/3); Single sideband = 16.67% (1/6) (B) Total sideband efficiency = 50%; Single sideband = 25% (C) Total sideband efficiency = 66.67%; Single sideband = 33.33% (D) Total sideband efficiency = 25%; Single sideband = 12.5%

Q.153 An RC envelope detector with time constant RC is used to demodulate an AM signal having carrier frequency fc and maximum message frequency fm (with modulation index μ). To avoid both carrier ripple and diagonal clipping, RC must satisfy: (A) 1/fc ≪ RC ≤ √(1 − μ2) / (2π μ fm) (B) RC ≪ 1/fc (C) RC ≫ 1/fm (D) RC = 1 / (fc + fm)

Q.154 Which amplitude modulation scheme is universally used for analog television video broadcasting to conserve bandwidth while avoiding the steep filter roll-off requirements of SSB near DC? (A) Standard full-carrier DSB-AM (B) Vestigial Sideband (VSB) modulation (C) Single Sideband (SSB-SC) via Hilbert transform (D) Wideband FM

Q.155 In Hilbert transform-based (phase-shift method) Single Sideband Suppressed Carrier (SSB-SC) generation for message m(t) with Hilbert transform m̂(t), the Upper Sideband (USB) and Lower Sideband (LSB) signals are given by: (A) sUSB(t) = m(t) cos(ωct) − m̂(t) sin(ωct); sLSB(t) = m(t) cos(ωct) + m̂(t) sin(ωct) (B) sUSB(t) = m(t) cos(ωct) + m̂(t) sin(ωct); sLSB(t) = m(t) cos(ωct) − m̂(t) sin(ωct) (C) sUSB(t) = m(t) sin(ωct) + m̂(t) cos(ωct) (D) sUSB(t) = [1 + m(t)] cos(ωct)

Q.156 A sinusoidal carrier is frequency-modulated by a 4 kHz audio tone, producing a peak frequency deviation Δf = 20 kHz. By Carson's rule, what are the modulation index β and the transmission bandwidth BW? (A) β = 5, BW = 48 kHz (B) β = 5, BW = 40 kHz (C) β = 0.2, BW = 48 kHz (D) β = 4, BW = 40 kHz

Q.157 In an FM signal s(t) = Ac cos[2π fc t + β sin(2π fm t)], if the modulation index β is increased from 1 to 5 while keeping Ac constant, the total average power of the FM signal: (A) Increases by a factor of 25 (B) Remains constant at Ac2 / 2 (C) Increases by (1 + β2/2) (D) Decreases as carrier Bessel function J0(β) drops

Q.158 In an FM broadcasting system, Pre-emphasis at the transmitter (and De-emphasis at the receiver) is used to improve the output SNR at high audio frequencies. The Pre-emphasis and De-emphasis circuits are essentially: (A) Pre-emphasis: High-Pass Filter (HPF); De-emphasis: Low-Pass Filter (LPF) (B) Pre-emphasis: Low-Pass Filter (LPF); De-emphasis: High-Pass Filter (HPF) (C) Both are Band-Stop Filters (D) Both are All-Pass Filters

Q.159 In a superheterodyne receiver tuned to a station at fc = 1000 kHz with an intermediate frequency IF = 455 kHz and high-side local oscillator injection (fLO = fc + IF = 1455 kHz), what is the image frequency fsi? (A) 545 kHz (B) 1910 kHz (C) 1455 kHz (D) 910 kHz

Q.160 A bandpass signal has a spectrum concentrated between fL = 20 kHz and fH = 30 kHz (bandwidth B = 10 kHz). According to the Bandpass Sampling Theorem, what is the minimum sampling frequency fs(min) for alias-free sampling? (A) 60 kHz (B) 20 kHz (C) 40 kHz (D) 10 kHz

Q.161 A voice signal bandlimited to 4 kHz is sampled at the Nyquist rate and encoded using an 8-bit uniform PCM system. What are the transmitted bit rate Rb and the approximate Signal-to-Quantization-Noise Ratio (SQNR) for a full-load sinusoidal input? (A) Rb = 64 kbps, SQNR ≈ 49.92 dB (B) Rb = 32 kbps, SQNR ≈ 48.0 dB (C) Rb = 64 kbps, SQNR ≈ 37.9 dB (D) Rb = 128 kbps, SQNR ≈ 49.92 dB

Q.162 In a linear Delta Modulation (DM) system with step size Δ and sampling frequency fs = 1/Ts, slope-overload distortion is avoided for a sinusoidal input m(t) = Am cos(2π fm t) if: (A) Δ ⋅ fs ≥ 2π fm Am (B) Δ ⋅ fs ≤ 2π fm Am (C) Δ ≥ Am (D) fs = 2 fm

Q.163 In non-Uniform quantization (companding) for speech PCM, which logarithmic companding law is standard in Europe/India (E1) vs. North America/Japan (T1)? (A) A-law in Europe/India; μ-law in North America/Japan (B) μ-law in Europe/India; A-law in North America/Japan (C) Linear quantization in both (D) Delta-Sigma law in both

Q.164 A binary data stream at bit rate Rb = 10 kbps is transmitted over a baseband channel using raised-cosine pulse shaping with roll-off factor α = 0.5. What is the required transmission bandwidth B? (A) 5.0 kHz (B) 7.5 kHz (C) 10.0 kHz (D) 15.0 kHz

Q.165 If a digital communication system transmits data at bit rate Rb = 24 Mbps using 16-QAM over an ideal Nyquist bandpass channel (roll-off α = 0), what are the symbol (baud) rate Rs and the minimum passband bandwidth B? (A) Rs = 6 Mbaud, B = 6 MHz (B) Rs = 6 Mbaud, B = 12 MHz (C) Rs = 4 Mbaud, B = 4 MHz (D) Rs = 24 Mbaud, B = 6 MHz

Q.166 What is the relation between the bit energy-to-noise spectral density ratio (Eb/N0) and the symbol energy-to-noise ratio (Es/N0) for an M-ary modulation scheme (where k = log2 M bits per symbol)? (A) Es/N0 = k ⋅ (Eb/N0) (B) Eb/N0 = k ⋅ (Es/N0) (C) Es/N0 = (Eb/N0) / M (D) Es/N0 = Eb/N0

Q.167 In an AWGN channel with two-sided noise PSD N0/2, a pulse g(t) of energy Eg is detected using a Matched Filter with impulse response h(t) = g(T − t). What is the maximum output Signal-to-Noise Ratio (SNRmax) at sampling instant t = T? (A) 2 Eg / N0 (B) Eg / N0 (C) Eg / (2 N0) (D) 4 Eg / N0

Q.168 Among coherent BPSK, QPSK, BFSK, and non-coherent DPSK operating at the same bit energy-to-noise ratio (Eb/N0) in an AWGN channel, which schemes achieve the lowest Bit Error Rate (BER)? (A) Coherent BPSK and QPSK (both have Pe = Q(√(2Eb/N0))) (B) Coherent BFSK (Pe = Q(√(Eb/N0))) (C) Non-coherent DPSK (D) OOK (ASK)

Q.169 A discrete memoryless source emits 4 symbols with probabilities {1/2, 1/4, 1/8, 1/8}. What is the Shannon entropy H(X) of the source in bits/symbol? (A) 1.50 bits/symbol (B) 1.75 bits/symbol (C) 2.00 bits/symbol (D) 1.25 bits/symbol

Q.170 An AWGN channel has bandwidth B = 4 kHz and signal-to-noise power ratio SNR = 15. According to the Shannon-Hartley theorem, what is the channel capacity C, and what is the absolute minimum Eb/N0 (Shannon limit) as B → ∞? (A) C = 16 kbps; Shannon limit (Eb/N0)min = ln(2) ≈ 0.693 (−1.59 dB) (B) C = 60 kbps; Shannon limit = 0 dB (C) C = 16 kbps; Shannon limit = 1 (0 dB) (D) C = 8 kbps; Shannon limit = −3 dB

Q.171 For two random variables X and Y, which identity correctly relates the Mutual Information I(X; Y) to the marginal and conditional entropies? (A) I(X; Y) = H(X) − H(X¦Y) = H(Y) − H(Y¦X) = H(X) + H(Y) − H(X, Y) (B) I(X; Y) = H(X) + H(X¦Y) (C) I(X; Y) = H(X, Y) − H(X) − H(Y) (D) I(X; Y) = H(X¦Y) + H(Y¦X)

Q.172 A linear block code has a minimum Hamming distance dmin = 5 between any two valid codewords. Up to how many bit errors can this code detect (ed), and up to how many bit errors can it correct (t)? (A) Detect up to 4 errors; Correct up to 2 errors (B) Detect up to 5 errors; Correct up to 2 errors (C) Detect up to 4 errors; Correct up to 3 errors (D) Detect up to 2 errors; Correct up to 1 error

Q.173 For a wide-sense stationary (WSS) random process X(t) with autocorrelation function RX(τ) = 9 + 4 e−2¦τ¦, what are the mean-square value (total average power), the DC power, and the AC power (variance σX2)? (A) Total power = 13, DC power = 9, AC power (variance) = 4 (B) Total power = 9, DC power = 4, AC power = 5 (C) Total power = 13, DC power = 3, AC power = 10 (D) Total power = 5, DC power = 9, AC power = 4

Q.174 In a Direct-Sequence Spread Spectrum (DS-SS) system, if the data bit rate is Rb = 10 kbps and the PN (pseudo-noise) chip rate is Rc = 10 Mcps, what is the Processing Gain PG in decibels (dB)? (A) 20 dB (B) 30 dB (C) 40 dB (D) 10 dB

Q.175 In GSM 2G cellular communication, which hybrid multiple-access scheme is used, and what is the primary advantage of CDMA over FDMA/TDMA? (A) GSM uses FDMA + TDMA; CDMA allows all users to share the same frequency band simultaneously using orthogonal/pseudo-random spreading codes (B) GSM uses pure CDMA; FDMA uses spreading codes (C) GSM uses OFDMA; CDMA requires guard times (D) GSM uses pure FDMA

Electromagnetics, Waveguides & Antennas

Q.176 For any vector field A and scalar field V, which pair of vector calculus identities is identically zero for all smooth fields? (A) ∇ ⋅ (∇ × A) = 0 and ∇ × (∇V) = 0 (B) ∇ ⋅ (∇V) = 0 and ∇ × (∇ × A) = 0 (C) ∇ × A = 0 and ∇ ⋅ A = 0 (D) ∇ ⋅ (∇ ⋅ A) = 0

Q.177 By Gauss's Law in differential form (∇ ⋅ D = ρv), if the electric flux density in a region is D = (2x ax + 3y2 ay + 4z az) C/m2, what is the volume charge density ρv at the point (1, 2, 3)? (A) 18 C/m3 (B) 20 C/m3 (C) 12 C/m3 (D) 26 C/m3

Q.178 In a charge-free dielectric region with uniform permittivity ε, the electrostatic potential V satisfies which partial differential equation? (A) Laplace's equation: ∇2V = 0 (B) Poisson's equation with non-zero source: ∇2V = −ρv/ε ≠ 0 (C) Helmholtz wave equation: ∇2V + k2V = 0 (D) Diffusion equation: ∇2V = ∂V/∂t

Q.179 Across a charge-free interface between two lossless dielectric media (ε1, ε2), which boundary conditions hold for the tangential electric field Et and normal electric flux density Dn? (A) Et1 = Et2 and Dn1 = Dn2 (i.e., ε1 En1 = ε2 En2) (B) Dt1 = Dt2 and En1 = En2 (C) Et1 = 0 and Dn1 = 0 (D) Et1 / Et2 = ε2 / ε1

Q.180 Why does the magnetic flux density B always satisfy ∇ ⋅ B = 0 (and ∮S B ⋅ dS = 0 over any closed surface)? (A) Because magnetic field lines are irrotational (B) Because isolated magnetic monopoles do not exist, so magnetic flux lines always form closed loops (C) Because permeability μ is constant in all materials (D) Because electric charges are stationary

Q.181 Maxwell modified Ampere's circuital law to ∇ × H = J + ∂D/∂t by introducing the displacement current density Jd = ∂D/∂t in order to make the equation consistent with: (A) Coulomb's law for static charges (B) The continuity equation of charge conservation (∇ ⋅ J = −∂ρv/∂t) for time-varying fields (C) Ohm's law in conductors (D) Biot-Savart law

Q.182 A uniform plane wave propagates in a lossless non-magnetic dielectric medium having relative permittivity εr = 4 and μr = 1. What are the phase velocity vp and the intrinsic impedance η of the medium? (Take c = 3 × 108 m/s, η0 = 120π ≈ 377 Ω) (A) vp = 1.5 × 108 m/s, η = 60π ≈ 188.5 Ω (B) vp = 3 × 108 m/s, η = 240π Ω (C) vp = 0.75 × 108 m/s, η = 30π Ω (D) vp = 1.5 × 108 m/s, η = 240π Ω

Q.183 If the skin depth of a good conductor at frequency f1 = 1 MHz is δ1 = 0.1 mm, what will be its skin depth δ2 at frequency f2 = 100 MHz? (A) 0.01 mm (B) 0.001 mm (C) 1.0 mm (D) 0.1 mm

Q.184 A uniform plane wave in free space (η0 = 120π Ω) has electric field E(z, t) = 120π cos(ωt − βz) ax V/m. What are the direction of propagation and the time-average Poynting vector Pavg? (A) +z direction; Pavg = 60π az W/m2 (B) −z direction; Pavg = 120π az W/m2 (C) +x direction; Pavg = 60π ax W/m2 (D) +z direction; Pavg = 120π az W/m2

Q.185 An electromagnetic plane wave propagating in the +z direction has electric field E(z, t) = E0 cos(ωt − βz) ax + E0 sin(ωt − βz) ay. Since the x and y components have equal amplitudes and a 90° phase difference, the wave polarization is: (A) Linearly polarized (B) Circularly polarized (C) Elliptically polarized with axial ratio 2 (D) Unpolarized

Q.186 A uniform plane wave is normally incident from air (η1 = 120π Ω) onto a lossless dielectric medium with εr2 = 9, μr2 = 1 (so η2 = 40π Ω). What are the voltage reflection coefficient Γ and transmission coefficient τ at the interface? (A) Γ = −0.5, τ = +0.5 (B) Γ = +0.5, τ = +1.5 (C) Γ = −0.8, τ = +0.2 (D) Γ = 0, τ = 1

Q.187 When an unpolarized plane wave is incident from medium 1 (εr1) onto lossless non-magnetic medium 2 (εr2) at the Brewster angle θB = tan−1√(εr2/εr1), which polarization component undergoes zero reflection? (A) Parallel (TM / p-polarized) component has zero reflection, so the reflected wave is purely Perpendicular (TE / s-polarized) (B) Perpendicular (TE / s-polarized) component has zero reflection (C) Both parallel and perpendicular components are totally reflected (D) The transmitted wave is completely extinguished

Q.188 A lossless transmission line has characteristic impedance Z0 = 50 Ω and is terminated in a resistive load ZL = 150 Ω. What are the magnitude of the reflection coefficient ¦Γ¦ and the Voltage Standing Wave Ratio (VSWR)? (A) ¦Γ¦ = 0.5, VSWR = 3 (B) ¦Γ¦ = 0.5, VSWR = 2 (C) ¦Γ¦ = 0.33, VSWR = 3 (D) ¦Γ¦ = 0.75, VSWR = 4

Q.189 What is the input impedance Zin of a lossless quarter-wave (l = λ/4) transmission line of characteristic impedance Z0 terminated in load impedance ZL, and what is the input impedance of a shorted λ/4 line (ZL = 0)? (A) Zin = Z02 / ZL; for a shorted λ/4 line (ZL = 0), Zin = ∞ (open circuit / parallel resonant tank) (B) Zin = ZL; for ZL = 0, Zin = 0 (C) Zin = √(Z0 ZL); for ZL = 0, Zin = Z0 (D) Zin = j Z0; for ZL = 0, Zin = j Z0

Q.190 To match a lossless transmission line of characteristic impedance Z0 = 50 Ω to a purely resistive load RL = 200 Ω using a quarter-wave (λ/4) matching transformer, the required characteristic impedance Z0T of the quarter-wave section is: (A) 100 Ω (B) 125 Ω (C) 75 Ω (D) 400 Ω

Q.191 On a standard normalized Smith Chart (impedance chart), what do the center of the chart (Γ = 0), the leftmost point (Γ = −1), and the rightmost point (Γ = +1) represent? (A) Center: Matched load (z = 1 + j0); Leftmost: Short circuit (z = 0); Rightmost: Open circuit (z = ∞) (B) Center: Short circuit; Leftmost: Matched load; Rightmost: Open circuit (C) Center: Open circuit; Leftmost: Short circuit; Rightmost: Matched load (D) Center: VSWR = ∞; Outer unit circle: VSWR = 1

Q.192 A transmission line with distributed parameters R, L, G, and C per unit length is Distortionless (attenuation α is independent of frequency and phase constant β is linear with ω) if Heaviside's condition holds: (A) R / L = G / C (B) R ⋅ L = G ⋅ C (C) R = 0 and C = 0 (D) R / G = L / C

Q.193 An air-filled rectangular waveguide has inner cross-sectional dimensions a = 3 cm and b = 1.5 cm (with a > b). What are its dominant mode and the cutoff frequency fc10 of the dominant mode? (Take c = 3 × 108 m/s) (A) Dominant mode: TE10; Cutoff frequency fc10 = 5 GHz (B) Dominant mode: TM11; Cutoff frequency fc = 10 GHz (C) Dominant mode: TE10; Cutoff frequency fc10 = 10 GHz (D) Dominant mode: TEM; Cutoff frequency fc = 0 Hz

Q.194 In an air-filled rectangular waveguide, why is the TM10 or TM01 mode non-existent, and what is the lowest-order TM mode that can propagate? (A) Boundary conditions require Ez = 0 on all four conducting walls, making Ez = 0 everywhere if m = 0 or n = 0; thus the lowest-order TM mode is TM11 (B) TM10 has infinite cutoff frequency; lowest TM mode is TM01 (C) TM00 is the dominant TM mode (D) TM modes cannot exist in rectangular waveguides

Q.195 In a lossless air-filled waveguide operating at frequency f = 10 GHz in a mode whose cutoff frequency is fc = 6 GHz, what are the phase velocity vp and group velocity vg in terms of the speed of light c? (A) vp = 1.25 c (5c/4) and vg = 0.80 c (4c/5), satisfying vp ⋅ vg = c2 (B) vp = 0.80 c and vg = 1.25 c (C) vp = c and vg = c (D) vp = 0.60 c and vg = 0.80 c

Q.196 For an antenna with maximum radiation directivity D and effective aperture area Ae operating at wavelength λ, the universal relationship between D and Ae, and the directivity of a half-wave (λ/2) dipole vs. anertzian (infinitesimal) dipole are: (A) D = 4π Ae / λ2; Hertzian dipole D = 1.5 (1.76 dBi), Half-wave dipole D = 1.64 (2.15 dBi, Rrad ≈ 73 Ω) (B) D = 2π Ae / λ2; Half-wave dipole D = 1.5 (Rrad = 50 Ω) (C) D = Ae / λ2; Hertzian dipole D = 1.0 (D) D = 4π λ2 / Ae; Half-wave dipole D = 3.0

Q.197 According to the Friis free-space transmission equation, if the distance R between a transmitting antenna and a receiving antenna is doubled AND the operating frequency f is also doubled (keeping antenna gains Gt and Gr constant), the received power Pr changes by: (A) Decreases by 6 dB (factor of 4) (B) Decreases by 12 dB (factor of 16) (C) Remains unchanged (D) Increases by 6 dB

Q.198 Two identical isotropic antennas are spaced by a distance d = λ/2 along the axis of the array and fed with currents of equal amplitude and 0° phase difference (in-phase). This configuration forms a: (A) Broadside array (maximum radiation perpendicular to the array axis, nulls along the array axis) (B) End-fire array (maximum radiation along the array axis) (C) Unidirectional cardioid array (D) Isotropic radiator

Q.199 A step-index optical fiber has a core refractive index n1 = 1.50 and a cladding refractive index n2 = 1.47 (launched from air with n0 = 1). What is the Numerical Aperture (NA) of the fiber? (A) √(1.502 − 1.472) = √0.0891 ≈ 0.298 (B) 1.50 − 1.47 = 0.030 (C) √(1.50 − 1.47) ≈ 0.173 (D) 1.47 / 1.50 = 0.980

Q.200 A police Doppler radar operating at carrier frequency f0 = 10 GHz (wavelength λ = 3 cm) tracks a vehicle moving directly toward the radar at speed v = 30 m/s (108 km/h). What is the Doppler frequency shift fd = 2v / λ observed in the reflected radar echo? (A) 1 kHz (B) 2 kHz (C) 500 Hz (D) 10 kHz

SECTION IX: HINTS, KEY FORMULAS & SHORTCUTS FOR SOLVING QUESTIONS (Q.1 – Q.200) Concise GATE & PSU exam shortcuts, key equations, and step-by-step derivations for rapid problem-solving

Hints & Shortcuts — Electronic Devices & VLSI

Q.1 [Ans: (B)]: Use mass-action law at thermal equilibrium: p0 = ni2 / ND = (1.5 × 1010)2 / 1016 = 2.25 × 104 cm−3. Q.2 [Ans: (B)]: Einstein relation: Dn / μn = kT / q = VT ⇒ Dn = 1000 × 0.0259 = 25.9 cm2/s. Q.3 [Ans: (B)]: Depletion width W ∝ 1/√N; higher doping narrows W and increases peak electric field for a given voltage, reducing VBR. Q.4 [Ans: (B)]: Breakdown below ~4 V occurs in heavily doped junctions via Zener tunneling and has a negative temperature coefficient. Q.5 [Ans: (B)]: Zener breakdown has a negative temperature coefficient, whereas Avalanche breakdown has a positive temperature coefficient (due to shorter mean free path at higher T).

Q.6 [Ans: (A)]: Diffusion capacitance CD = (τ ⋅ ID) / (η VT), so CD ∝ τ ⋅ ID. Q.7 [Ans: (A)]: For an abrupt junction, Cj ∝ (Vbi + VR)−1/2; for a linearly graded junction, Cj ∝ (Vbi + VR)−1/3. Q.8 [Ans: (B)]: Use λ (in μm) = 1.24 / Eg(eV) = 1.24 / 1.55 = 0.80 μm (800 nm). Q.9 [Ans: (C)]: Silicon (Si) and Germanium (Ge) are indirect bandgap semiconductors where radiative recombination requires phonon assistance. Q.10 [Ans: (B)]: Widening the depletion region via the intrinsic layer lowers Cj = εA/W (faster speed) and captures more incident photons.

Q.11 [Ans: (B)]: Thermal generation/recombination of minority carriers is slow, so at high frequency the inversion charge cannot follow the AC gate signal and C stays at Cmin. Q.12 [Ans: (B)]: Cox = εox ε0 / tox = (3.9 × 8.854 × 10−14) / (10 × 10−7 cm) ≈ 3.45 × 10−7 F/cm2 = 345 nF/cm2. Q.13 [Ans: (B)]: Strong inversion occurs when the surface electron concentration equals the bulk hole concentration, which requires φs = 2φF. Q.14 [Ans: (B)]: Since VGS = 3 V > VTH and VDS = 1.5 V < (VGS − VTH = 2 V), the device is in the Linear (Triode) region. Q.15 [Ans: (B)]: Since ID = (1/2) kn VOV2, differentiating with respect to VGS gives gm = kn VOV = 2ID / VOV.

Q.16 [Ans: (B)]: Channel-length modulation reduces Leff with increasing VDS in saturation, giving the MOSFET a finite output resistance ro = 1 / (λ ID). Q.17 [Ans: (B)]: VTH = VTH0 + γ(√(2φF + VSB) − √(2φF)); increasing VSB widens the bulk depletion region and increases VTH. Q.18 [Ans: (B)]: Base-width modulation (Early effect) causes collector current IC to increase slightly with VCE, defined by the Early voltage VA. Q.19 [Ans: (A)]: β = α / (1 − α) = 0.98 / 0.02 = 49, and γ = 1 + β = 50. Q.20 [Ans: (D)]: Both junctions reverse-biased = Cut-off; both forward-biased = Saturation; EB forward & CB reverse = Forward-active.

Q.21 [Ans: (B)]: Dry oxidation is slower but yields superior, high-density oxide with low interface trap density, making it ideal for thin MOSFET gate oxides. Q.22 [Ans: (B)]: Ion implantation controls dopant dose via beam current/time and depth via acceleration energy at low temperature, followed by annealing. Q.23 [Ans: (C)]: Hall coefficient RH = −1 / (n⋅q) < 0 for n-type (electrons) and RH = +1 / (p⋅q) > 0 for p-type (holes). Q.24 [Ans: (B)]: Maximum power Pmax = FF × Voc × Isc = 0.75 × 0.6 × 2 = 0.90 W. Q.25 [Ans: (B)]: Schottky diodes are majority-carrier devices with no minority-carrier charge storage, giving them ultra-fast switching and low forward drop (~0.3 V).

Hints & Shortcuts — Analog Electronics

Q.26 [Ans: (A)]: Half-wave rectifier has PIV = Vm and ripple factor = 1.21; full-wave bridge has PIV = Vm and ripple factor = 0.482. Q.27 [Ans: (C)]: In a center-tapped full-wave rectifier (without or with capacitor filter), PIV = 2Vm = 2 × 10 = 20 V. Q.28 [Ans: (B)]: A positive clamper clamps the negative peak (−5 V) to 0 V while preserving the 10 V peak-to-peak swing, so the output swings from 0 V to +10 V. Q.29 [Ans: (B)]: Source current IS = (Vin − VZ) / RS = (10 − 5)/100 = 50 mA. Thus IL(max) = IS − IZK = 50 − 2 = 48 mA. Q.30 [Ans: (C)]: Voltage-divider bias with emitter degeneration resistor RE yields stability factor S(ICO) ≈ 1 + RB/RE, making IC nearly independent of β.

Q.31 [Ans: (A)]: gm = ICQ / VT = 2.6 mA / 26 mV = 100 mS; rπ = β / gm = 100 / 0.1 = 1000 Ω = 1 kΩ. Q.32 [Ans: (B)]: Emitter degeneration introduces negative current-series feedback, raising Rin = rπ + (1 + β)RE and stabilizing ¦Av¦ ≈ RC / RE. Q.33 [Ans: (C)]: The Common-Collector (Emitter Follower) acts as a voltage buffer with high Rin, low Rout, and Av ≈ 1. Q.34 [Ans: (B)]: In a Cascode stage, the collector of the input CE transistor sees the low input impedance (1/gm) of the CB stage, reducing CE voltage gain to ~−1 and suppressing Miller multiplication of Cμ. Q.35 [Ans: (D)]: Miller input capacitance: CM(in) = Cf(1 − Av) = 2 pF × (1 − (−99)) = 2 × 100 = 200 pF.

Q.36 [Ans: (C)]: CMRR = ¦Ad / Acm¦ = 2000 / 0.02 = 105. In dB, CMRRdB = 20 log10(105) = 100 dB. Q.37 [Ans: (C)]: Class-A (series-fed): 25%; Class-A (transformer-coupled): 50%; Class-B push-pull: π/4 ≈ 78.5%. Q.38 [Ans: (B)]: Class-AB biasing pre-biases both output transistors just at the edge of conduction to eliminate the dead-zone around zero crossings. Q.39 [Ans: (A)]: Series mixing at the input increases Rin by (1 + Aβ); voltage (shunt) sampling at the output decreases Rout by (1 + Aβ). Q.40 [Ans: (A)]: Desensitivity factor 1 + Aβ = 1 + 1000(0.009) = 10. Thus Af = A / 10 = 100, and fHf = fH × 10 = 100 kHz (gain-bandwidth product remains constant).

Q.41 [Ans: (C)]: Non-inverting gain Av = 1 + Rf/R1 = 1 + 18/2 = 10. Thus Vout = 10 × 0.5 V = +5.0 V. Q.42 [Ans: (A)]: For no slew-rate limiting: 2π f Vm ≤ SR ⇒ Vm ≤ SR / (2π f) = (2 × 106) / (2π × 105) = 10 / π V ≈ 3.18 V. Q.43 [Ans: (B)]: Positive feedback creates a hysteresis dead-band (VH = VUTP − VLTP) that prevents false multiple triggering from noisy inputs. Q.44 [Ans: (B)]: Sustained oscillation requires unity loop gain magnitude ¦Aβ¦ = 1 and zero (or integer multiple of 360°) total loop phase shift. Q.45 [Ans: (A)]: For a 3-stage RC network, β = −1/29 at f0 = 1 / (2π√6 RC) (or √6 / (2π RC) depending on high-pass/low-pass topology), requiring ¦A¦ ≥ 29.

Q.46 [Ans: (A)]: The Wien lead-lag network has zero phase shift and attenuation β = 1/3 at f0 = 1/(2π RC), so non-inverting gain must be 1 + Rf/R1 = 3. Q.47 [Ans: (A)]: Equivalent capacitance Ceq = C1C2/(C1+C2) = 100 pF. Then √(L Ceq) = √(10−4 × 10−10) = 10−7 s ⇒ f0 = 1/(2π × 10−7) ≈ 1.59 MHz. Q.48 [Ans: (B)]: Three equal 5 kΩ resistors divide VCC into (2/3) VCC (Threshold comparator reference) and (1/3) VCC (Trigger comparator reference). Q.49 [Ans: (B)]: In astable mode, the capacitor charges through (RA + RB) from (1/3)VCC to (2/3)VCC and discharges through RB from (2/3)VCC to (1/3)VCC. Q.50 [Ans: (B)]: Butterworth has a maximally flat magnitude response in the passband; Chebyshev has equiripple in the passband; Bessel has maximally flat group delay.

Hints & Shortcuts — Network Theory

Q.51 [Ans: (A)]: Number of independent KCL (nodal) equations = N − 1 = 5; number of independent KVL (fundamental loop/mesh) equations = B − N + 1 = 10 − 6 + 1 = 5. Q.52 [Ans: (C)]: For balanced Δ-to-Y conversion: RY = RΔ / 3 = 9 / 3 = 3 Ω (and RΔ = 3 RY). Q.53 [Ans: (B)]: Dependent sources cannot be deactivated because their values depend on circuit variables; deactivate independent sources and apply a test source Vtest/Itest. Q.54 [Ans: (B)]: When both RL and XL are variable, maximum power transfer requires complex conjugate matching: ZL = ZTh* = (6 − j8) Ω. Q.55 [Ans: (C)]: When the load is restricted to a pure resistance RL, maximum power transfer occurs at RL = ¦ZTh¦ = √(62 + 82) = 10 Ω.

Q.56 [Ans: (A)]: Reciprocity requires Z12 = Z21, Y12 = Y21, h12 = −h21, and AD − BC = 1. (Symmetry requires Z11 = Z22 and A = D.) Q.57 [Ans: (C)]: Series-series → add Z matrices; Parallel-parallel → add Y matrices; Cascade → multiply ABCD (transmission) matrices. Q.58 [Ans: (A)]: For a T-network: Z11 = Ra + Rc = 2 + 5 = 7 Ω, Z22 = Rb + Rc = 3 + 5 = 8 Ω, and Z12 = Z21 = Rc = 5 Ω. Q.59 [Ans: (A)]: At t = 0+, uncharged capacitor acts as a short circuit: i(0+) = 10 V / 500 kΩ = 20 μA. Time constant τ = RC = 1 s, so vC(1) = 10(1 − e−1) ≈ 6.32 V. Q.60 [Ans: (B)]: At t = 0+, iL(0+) = iL(0−) = 0 A (inductor acts as open circuit), so vL(0+) = 12 V. At t → ∞, inductor acts as short circuit, so i(∞) = 12/4 = 3 A.

Q.61 [Ans: (B)]: Since VL(s) = s L I(s) − L I0, the s-domain series model is an impedance sL in series with a voltage source L I0 aiding I(s). Q.62 [Ans: (A)]: ω0 = 1/√(LC) = 1/√(10−8) = 104 rad/s; Q = ω0L/R = (104 × 10−2)/10 = 10; BW = R/L = ω0/Q = 1000 rad/s. Q.63 [Ans: (B)]: At series resonance, VL and VC are equal in magnitude and 180° out of phase, each magnified by Q: VC = Q × Vsource = 25 × 10 = 250 V. Q.64 [Ans: (B)]: In a parallel RLC circuit above ω0, capacitive susceptance ωC > 1/(ωL), so the circuit is net capacitive and supply current leads voltage. Q.65 [Ans: (B)]: For series-opposing connection: Leq = L1 + L2 − 2M = 4 + 9 − 2(3) = 7 H (series-aiding would be L1 + L2 + 2M = 19 H).

Q.66 [Ans: (A)]: Cube resistance shortcuts: Body diagonal = (5/6)R; Face diagonal = (3/4)R; Single edge = (7/12)R. Q.67 [Ans: (B)]: Tellegen's theorem (conservation of instantaneous power) depends solely on network topology (KVL and KCL), independent of element characteristics. Q.68 [Ans: (A)]: For an R–C driving-point impedance ZRC(s), poles and zeros alternate on the negative real axis with a pole (at s = 0 or closest to origin) first, and Z(∞) ≤ Z(0). Q.69 [Ans: (A)]: Irms = √(I02 + I1,rms2) = √(32 + (4√2 / √2)2) = √(9 + 16) = 5 A. Q.70 [Ans: (A)]: Apparent power S = √(P2 + Q2) = √(42 + 32) = 5 kVA; power factor = P/S = 4/5 = 0.8 lagging (since Q is inductive).

Q.71 [Ans: (A)]: In Star (Y): Iph = IL and Vph = VL/√3. In Delta (Δ): Vph = VL and Iph = IL/√3. Q.72 [Ans: (B)]: A dual-impedance network satisfies Z1 Z2 = R02 (so √(L/C) = R0), making its input impedance equal to R0 at all frequencies. Q.73 [Ans: (A)]: L{tn u(t) / n!} = 1 / sn+1. Thus L{t u(t)} = 1/s2 and L{(t2/2) u(t)} = 1/s3. Q.74 [Ans: (A)]: i(0+) = lims→∞ s I(s) = 4; i(∞) = lims→0 s I(s) = 10 / 2 = 5. Q.75 [Ans: (A)]: Every spanning tree of an N-node connected graph has (N − 1) twigs; the remaining (B − N + 1) branches are links (chords).

Hints & Shortcuts — Signals & Systems and DSP

Q.76 [Ans: (B)]: E = ∫0∞ ¦2 e−4t¦2 dt = 4 ∫0∞ e−8t dt = 4 / 8 = 0.50 J. Q.77 [Ans: (A)]: For orthogonal frequencies, average power P = IDC2 + (A12/2) + (A22/2) = 42 + (36/2) + (64/2) = 16 + 18 + 32 = 66 W. Q.78 [Ans: (B)]: Superposition holds so it is Linear; multiplying by explicit time t makes it Time-Varying; depending only on (t − 2) makes it Causal. Q.79 [Ans: (B)]: Convolving two unequal rectangular pulses of widths T1 and T2 produces a trapezoid of total duration T1 + T2 = 8 s (with flat top of width ¦T2 − T1¦ = 2 s). Q.80 [Ans: (B)]: By the sifting (sampling) property of the Dirac delta function: ∫ f(t) δ(t − t0) dt = f(t0) = 22 + 3(2) + 5 = 15.

Q.81 [Ans: (A)]: Since Σn=0∞ ¦h[n]¦ = 1 / (1 − 0.5) = 2 < ∞, the system is BIBO stable and its DC gain is H(ej0) = Σ h[n] = 2. Q.82 [Ans: (B)]: A real and even time-domain signal always has a real and even spectrum (both in Fourier series and Fourier transform). Q.83 [Ans: (B)]: Since a sinc pulse A sinc(Wt) has a rectangular spectrum of height A/W and width W, its energy is E = A2 / W = 42 / 2 = 8 J. Q.84 [Ans: (A)]: Time compression by factor a expands the spectrum by a and scales amplitude by 1/¦a¦: FT{x(at)} = (1/¦a¦) X(f/a). Q.85 [Ans: (B)]: L{cos(3t) u(t)} = s / (s2 + 9); applying s-domain shifting (e−2t ↔ s → s + 2) gives (s + 2) / [(s + 2)2 + 9].

Q.86 [Ans: (B)]: For BIBO stability in continuous time, the ROC must include the jω-axis (Re(s) = 0). Thus ROC is −1 < Re(s) < 3, corresponding to a two-sided (non-causal) impulse response. Q.87 [Ans: (B)]: The highest frequency in y(t) = x(t)⋅x2(t) is fmax = f1,max + f2,max = 250 Hz + 150 Hz = 400 Hz. Thus Nyquist rate fN = 2 fmax = 800 Hz. Q.88 [Ans: (B)]: Since f0 > fs/2, aliasing occurs. The aliased frequency inside [0, fs/2] is falias = ¦f0 − fs¦ = ¦1200 − 1000¦ = 200 Hz. Q.89 [Ans: (B)]: Both an u[n] and −an u[−n − 1] share the exact same algebraic Z-transform 1/(1 − az−1), distinguished solely by their ROCs: ¦z¦ > ¦a¦ vs. ¦z¦ < ¦a¦. Q.90 [Ans: (A)]: Differentiating X(z) = Σ x[n] z−n gives dX(z)/dz = Σ (−n) x[n] z−n−1, so Z{n x[n]} = −z dX(z)/dz.

Q.91 [Ans: (B)]: Linear convolution length is L = N1 + N2 − 1 = 3 + 4 − 1 = 6; thus N ≥ 6 prevents time-domain aliasing in circular convolution. Q.92 [Ans: (B)]: For a real N-point sequence, X[N − k] = X*[k]. For N = 8 and k = 2, X[6] = X*[2] = 3 + j4. Q.93 [Ans: (A)]: Radix-2 FFT requires (N/2) log2(N) = 512 × 10 = 5,120 complex multiplications, compared to N2 = 1,048,576 for direct DFT. Q.94 [Ans: (C)]: A linear-phase FIR filter of length M has constant group delay τg = (M − 1) / 2 = (5 − 1) / 2 = 2 samples. Q.95 [Ans: (B)]: Bilinear transform is a one-to-one conformal mapping that maps Re(s) < 0 inside ¦z¦ < 1 (and jω-axis onto ¦z¦ = 1 with frequency warping).

Q.96 [Ans: (B)]: Tapering windows (Hamming, Hanning, Blackman, Kaiser) trade a wider main lobe (wider transition band) for much lower side lobes (higher stopband attenuation). Q.97 [Ans: (B)]: An all-pass system of the form (z−1 − a*)/(1 − a z−1) has poles and zeros at conjugate-reciprocal locations and unit magnitude response for all ω. Q.98 [Ans: (B)]: A minimum-phase system is both causal/stable AND has a causal/stable inverse, requiring both poles and zeros to lie inside the stability region. Q.99 [Ans: (B)]: Setting ω0 N0 = (3π/7) N0 = 2π k ⇒ N0 = 14 k / 3. The smallest positive integer k is k = 3, giving N0 = 14. Q.100 [Ans: (B)]: Downsampling by M expands the discrete-time spectrum by M; to prevent aliasing, an anti-aliasing LPF with cutoff π/M must precede the downsampler.

Hints & Shortcuts — Control Systems

Q.101 [Ans: (B)]: Mason's determinant: Δ = 1 − (sum of all individual loop gains) + (sum of gain products of all pairs of non-touching loops) = 1 − (L1 + L2 + L3) + L1L3. Q.102 [Ans: (A)]: Comparing with 1/(τs + 1), time constant τ = 5 s; 2% settling time ts ≈ 4τ = 20 s (and 5% settling time ≈ 3τ = 15 s). Q.103 [Ans: (A)]: Comparing with ωn2 / (s2 + 2ζωns + ωn2): ωn = √100 = 10 rad/s, and 2ζ(10) = 12 ⇒ ζ = 0.6 (underdamped). Q.104 [Ans: (A)]: ωd = ωn√(1 − ζ2) = 5√(1 − 0.36) = 5 × 0.8 = 4 rad/s; peak time tp = π / ωd = π/4 s. Q.105 [Ans: (B)]: %Mp = 100 × e−πζ / √(1 − ζ^{2)}. For ζ = 1/√2, ζ / √(1 − ζ2) = 1, so %Mp = 100 × e−π ≈ 4.32% (for ζ = 0.5, %Mp ≈ 16.3%).

Q.106 [Ans: (A)]: One pole at origin s = 0 ⇒ Type 1; Kv = lims→0 s G(s) = 20 / (2 × 5) = 2; ramp steady-state error ess = 1 / Kv = 0.5. Q.107 [Ans: (B)]: A Type-2 system (two poles at s = 0) tracks step and ramp inputs with zero steady-state error, and tracks a parabolic input with constant error ess = 1/Ka. Q.108 [Ans: (C)]: For a 3rd-order polynomial, stability requires a1a2 > a0a3. Here a1a2 = 2 × 4 = 8 < a0a3 = 24, causing the s1 row entry (8 − 24)/2 = −8 < 0 and 2 sign changes in the first column ⇒ 2 RHP roots. Q.109 [Ans: (B)]: Routh conditions: (1) 6 + K > 0 ⇒ K > −6; (2) a1a2 > a0a3 ⇒ 6 × 11 > 1 × (6 + K) ⇒ 66 > 6 + K ⇒ K < 60. Thus −6 < K < 60. Q.110 [Ans: (B)]: An all-zero row in the Routh table occurs when the polynomial contains an even factor (s2 + ω2 or s2 − σ2) with roots symmetric about the origin.

Q.111 [Ans: (A)]: Number of asymptotes = P − Z = 4 − 1 = 3, so angles are (2q + 1)180°/3 = 60°, 180°, 300°. Centroid σA = (Σ poles − Σ zeros)/(P − Z) = [(0 − 1 − 2 − 5) − (−3)] / 3 = −5/3. Q.112 [Ans: (B)]: From K = −(s2 + 4s), setting dK/ds = −(2s + 4) = 0 gives the breakaway point at s = −2 (halfway between the two poles at 0 and −4). Q.113 [Ans: (C)]: Each pole contributes −20 dB/decade (−6 dB/octave) and each zero contributes +20 dB/decade; for P − Z = 3, net high-frequency slope is −60 dB/decade. Q.114 [Ans: (C)]: Initial low-frequency Bode magnitude slope = −20 × N dB/decade, where N is the number of poles at the origin (System Type). Thus −40 dB/decade ⇒ Type 2. Q.115 [Ans: (B)]: Phase Margin PM = 180° + ∠G(jωgc)H(jωgc) = 180° + (−135°) = +45°.

Q.116 [Ans: (B)]: Nyquist criterion: Z = N + P (where N is CW encirclements of −1+j0). For stability (Z = 0) with P = 1, we need N = −1 (1 counter-clockwise encirclement). Q.117 [Ans: (B)]: Since ¦e−jωT¦ = 1 and ∠e−jωT = −ωT, dead time leaves the Bode magnitude plot unchanged while adding linearly increasing phase lag, reducing PM. Q.118 [Ans: (B)]: For 0 < α < 1, the zero (−1/τ) lies to the right of the pole (−1/ατ), contributing positive phase (phase lead) with sin(φm) = (1 − α)/(1 + α). Q.119 [Ans: (B)]: Phase-Lead acts like PD control (adds phase lead, increases bandwidth, speeds up response); Phase-Lag acts like PI control (boosts low-frequency gain, reduces steady-state error). Q.120 [Ans: (B)]: PD control adds a zero (Kp + Kds), which adds damping (2ζωn increases by Kd) and reduces overshoot, though it amplifies high-frequency noise.

Q.121 [Ans: (B)]: Taking Laplace transform with zero initial conditions: sX(s) = AX(s) + BU(s) ⇒ X(s) = (sI − A)−1BU(s) ⇒ Y(s)/U(s) = C(sI − A)−1B + D. Q.122 [Ans: (A)]: Characteristic equation det(sI − A) = s(s + 5) + 6 = s2 + 5s + 6 = (s + 2)(s + 3) = 0 ⇒ eigenvalues are s = −2, −3. Q.123 [Ans: (B)]: Complete state controllability requires rank([B AB ... An−1B]) = n; complete observability requires rank([CT ATCT ... (AT)n−1CT]) = n. Q.124 [Ans: (B)]: A transfer function is a minimal realization (both controllable and observable) if and only if there are no pole-zero cancellations in C(sI − A)−1B. Q.125 [Ans: (B)]: By definition of matrix exponential: Φ(0) = eA⋅0 = I, Φ(t1+t2) = Φ(t1)Φ(t2), and Φ−1(t) = Φ(−t).

Hints & Shortcuts — Digital Electronics & 8085

Q.126 [Ans: (A)]: n-bit 2's complement range is −2n−1 to +(2n−1 − 1) = −128 to +127. Since +25 = 00011001, inverting bits (11100110) + 1 gives −25 = 11100111. Q.127 [Ans: (A)]: Binary-to-Gray: G3 = B3 = 1; G2 = B3 ⊕ B2 = 1 ⊕ 1 = 0; G1 = B2 ⊕ B1 = 1 ⊕ 0 = 1; G0 = B1 ⊕ B0 = 0 ⊕ 1 = 1 ⇒ 1011. Q.128 [Ans: (B)]: In A⋅B + A̅⋅C + B⋅C, the term B⋅C is the consensus (redundant) term of A⋅B and A̅⋅C, so F simplifies to A⋅B + A̅⋅C. Q.129 [Ans: (A)]: For n variables, total Boolean functions = 22^{n} = 216 = 65,536; number of self-dual functions = 22^{n−1} = 28 = 256. Q.130 [Ans: (B)]: Minimum 2-input NAND gates for XOR = 4; for XNOR = 5. Minimum 2-input NOR gates for XNOR = 4; for XOR = 5.

Q.131 [Ans: (A)]: Minterms (0, 2, 8, 10) form a 4-cell corner group B̅⋅D̅, and minterms (5, 7, 13, 15) form a 4-cell middle group B⋅D, giving F = B⋅D + B̅⋅D̅. Q.132 [Ans: (C)]: Stage 1 needs 64/4 = 16 MUXes; Stage 2 needs 16/4 = 4 MUXes; Stage 3 needs 4/4 = 1 MUX. Total = 16 + 4 + 1 = 21 (or (64 − 1)/(4 − 1) = 21). Q.133 [Ans: (B)]: In RCA, carry ripples sequentially through n stages (O(n) delay); CLA computes Ci+1 = Gi + PiCi in parallel via two-level logic. Q.134 [Ans: (A)]: CMOS has near-zero static power dissipation; ECL prevents transistor saturation (operating in active/cut-off only) to achieve sub-nanosecond propagation delay. Q.135 [Ans: (B)]: Dynamic power Pdyn = CL VDD2 f = 5 × 10−12 × (2)2 × 108 = 2 × 10−3 W = 2 mW.

Q.136 [Ans: (B)]: Edge triggering or Master-Slave configuration isolates the output from the input during the active clock level, preventing multiple toggles in a single clock cycle. Q.137 [Ans: (A)]: JK flip-flop: Q+ = JQ̅ + K̅Q; T flip-flop: Q+ = T ⊕ Q = TQ̅ + T̅Q; D flip-flop: Q+ = D. Q.138 [Ans: (A)]: Minimum clock period Tmin = tcq + tcomb + tsu = 3 + 12 + 5 = 20 ns ⇒ fmax = 1 / 20 ns = 50 MHz. Q.139 [Ans: (A)]: A MOD-10 counter cycles through 10 states (0000 to 1001). In an n-stage ripple counter, worst-case delay is n × tpd = 4 × 15 = 60 ns. Q.140 [Ans: (A)]: An n-bit Ring counter is a MOD-n counter (6 states, fout = 120/6 = 20 kHz); an n-bit Johnson counter is a MOD-2n counter (12 states, fout = 120/12 = 10 kHz).

Q.141 [Ans: (A)]: Moore output = f(Present State); Mealy output = f(Present State, Present Input), allowing Mealy machines to respond one clock cycle earlier with fewer states. Q.142 [Ans: (A)]: Step size = VFS / (28 − 1) = 5.10 V / 255 = 0.02 V = 20 mV. For input 128, Vout = 128 × 20 mV = 2.56 V. Q.143 [Ans: (A)]: Flash ADC converts in 1 step using 2n − 1 comparators (fastest); Dual-Slope ADC averages the input over an integration period, rejecting 50/60 Hz hum (highest accuracy/noise rejection). Q.144 [Ans: (B)]: An n-bit SAR ADC requires n clock cycles (vs. up to 2n − 1 cycles for a counter ADC), so Tconv = 8 × 1 μs = 8 μs. Q.145 [Ans: (A)]: Number of chips = (64K / 16K) × (16 / 8) = 4 × 2 = 8 chips. Address lines for 64K (= 216) words = 16 address lines.

Q.146 [Ans: (A)]: PROM: Fixed AND (decoder) + Programmable OR; PAL: Programmable AND + Fixed OR; PLA: Programmable AND + Programmable OR. Q.147 [Ans: (A)]: 8085 has 16 address lines (64 KB memory space) and 8 data lines; AD0–AD7 are time-multiplexed, and ALE goes high during T1 to latch A0–A7. Q.148 [Ans: (A)]: TRAP is the highest-priority, non-maskable interrupt (both edge and level sensitive) with vector address 4.5 × 8 = 36 = 0024H. Q.149 [Ans: (A)]: LHLD addr requires 5 machine cycles: Opcode Fetch (4T) + Operand Low Read (3T) + Operand High Read (3T) + Memory Read L (3T) + Memory Read H (3T) = 16 T-states. Q.150 [Ans: (A)]: In 8085, the stack grows downward in memory: a PUSH or CALL decrements SP by 2 (storing high byte at SP−1, low byte at SP−2), while POP or RET increments SP by 2.

Hints & Shortcuts — Communication Systems

Q.151 [Ans: (A)]: Total AM power Pt = Pc(1 + μ2/2) = 10 × (1 + 0.36/2) = 10 × 1.18 = 11.8 kW. Q.152 [Ans: (A)]: η = μ2 / (2 + μ2). At μ = 1, η = 1/3 = 33.33% in both sidebands combined, or 1/6 = 16.67% per sideband. Q.153 [Ans: (A)]: RC must be much larger than the carrier period 1/fc (to filter out RF) but small enough to follow the steepest envelope decay: RC ≤ √(1 − μ2) / (2π μ fm). Q.154 [Ans: (B)]: Video signals have high energy near DC and wide bandwidth; VSB transmits one full sideband plus a small vestige of the other sideband, allowing gradual filter roll-off. Q.155 [Ans: (A)]: Minus sign (−) gives Upper Sideband (USB); plus sign (+) gives Lower Sideband (LSB).

Q.156 [Ans: (A)]: Modulation index β = Δf / fm = 20 / 4 = 5; Carson's rule bandwidth BW = 2(Δf + fm) = 2(β + 1)fm = 2(20 + 4) = 48 kHz. Q.157 [Ans: (B)]: FM is a constant-envelope angle modulation scheme; its total average power P = Ac2 / 2 is independent of the modulation index β. Q.158 [Ans: (A)]: FM demodulator noise PSD rises parabolically (∝ f2) at high frequencies; Pre-emphasis boosts high-frequency audio (HPF) before transmission, and De-emphasis attenuates high-frequency noise (LPF) at the receiver. Q.159 [Ans: (B)]: For high-side injection (fLO = fc + IF), image frequency is fsi = fc + 2⋅IF = 1000 + 2(455) = 1910 kHz, which is rejected by the RF preselector stage prior to the mixer. Q.160 [Ans: (B)]: Bandpass sampling: fs(min) = 2 fH / k, where k = floor(fH / B) = floor(30 / 10) = 3. Thus fs(min) = 2(30) / 3 = 20 kHz (= 2B).

Q.161 [Ans: (A)]: Nyquist rate fs = 2 × 4 kHz = 8 kHz; Rb = n fs = 8 × 8 = 64 kbps; sinusoidal SQNR = 1.76 + 6.02n = 1.76 + 6.02(8) = 49.92 dB (adding 1 bit adds ~6 dB). Q.162 [Ans: (A)]: Maximum slope of the staircase tracker is Δ / Ts = Δ ⋅ fs, which must be ≥ maximum input slope ¦dm/dt¦max = 2π fm Am. Q.163 [Ans: (A)]: Europe/India ITU-T standard uses A-law companding (A ≈ 87.6); North America/Japan uses μ-law companding (μ = 255). Q.164 [Ans: (B)]: Baseband raised-cosine bandwidth: B = (Rb / 2)(1 + α) = (10 / 2)(1 + 0.5) = 7.5 kHz (with minimum Nyquist bandwidth Bmin = Rb/2 = 5 kHz for α = 0). Q.165 [Ans: (A)]: For M = 16 = 24 (4 bits/symbol), symbol rate Rs = Rb / log2(M) = 24 / 4 = 6 Mbaud. Minimum passband Nyquist bandwidth (α = 0) is B = Rs = 6 MHz.

Q.166 [Ans: (A)]: Since each M-ary symbol carries k = log2(M) bits, symbol energy is Es = k Eb, so Es/N0 = (log2 M)(Eb/N0). Q.167 [Ans: (A)]: A matched filter maximizes peak pulse SNR in white Gaussian noise; the maximum output SNR depends only on signal energy and noise PSD: SNRmax = 2 Eg / N0. Q.168 [Ans: (A)]: Coherent BPSK and QPSK share the exact same bit error probability Pe = Q(√(2Eb/N0)), which is 3 dB better in Eb/N0 than coherent orthogonal BFSK. Q.169 [Ans: (B)]: H(X) = −Σ pi log2(pi) = (1/2)(1) + (1/4)(2) + (1/8)(3) + (1/8)(3) = 0.5 + 0.5 + 0.375 + 0.375 = 1.75 bits/symbol. Q.170 [Ans: (A)]: C = B log2(1 + SNR) = 4 × log2(16) = 16 kbps. As B → ∞, reliable communication requires Eb/N0 > ln(2) ≈ −1.59 dB.

Q.171 [Ans: (A)]: Mutual information is symmetric and non-negative: I(X; Y) = H(X) − H(X¦Y) = H(X) + H(Y) − H(X, Y) ≥ 0. Q.172 [Ans: (A)]: Error detection capability: ed = dmin − 1 = 4; Error correction capability: t = floor((dmin − 1)/2) = floor(4/2) = 2. Q.173 [Ans: (A)]: Total average power E[X2(t)] = RX(0) = 9 + 4 = 13; DC power = (E[X])2 = RX(∞) = 9; Variance (AC power) σX2 = RX(0) − RX(∞) = 4. Q.174 [Ans: (B)]: Processing Gain PG = Rc / Rb = 10 × 106 / 104 = 1000. In dB, PGdB = 10 log10(1000) = 30 dB. Q.175 [Ans: (A)]: GSM divides each 200 kHz FDMA carrier into 8 TDMA time slots; CDMA separates users in code space across the entire bandwidth simultaneously.

Hints & Shortcuts — Electromagnetics, Waveguides & Antennas

Q.176 [Ans: (A)]: The divergence of the curl of any vector field is always zero (∇⋅(∇×A) = 0), and the curl of the gradient of any scalar field is always zero (∇×(∇V) = 0). Q.177 [Ans: (A)]: ρv = ∇ ⋅ D = ∂(2x)/∂x + ∂(3y2)/∂y + ∂(4z)/∂z = 2 + 6y + 4 = 6 + 6y. At (1, 2, 3), ρv = 6 + 6(2) = 18 C/m3. Q.178 [Ans: (A)]: Poisson's equation is ∇2V = −ρv/ε; in a charge-free region (ρv = 0), it reduces to Laplace's equation ∇2V = 0. Q.179 [Ans: (A)]: Tangential E is continuous (Et1 = Et2), and normal D is continuous across a charge-free boundary (Dn1 = Dn2), giving tan(α1)/tan(α2) = ε1/ε2. Q.180 [Ans: (B)]: Gauss's law for magnetism (∇ ⋅ B = 0) expresses the non-existence of isolated magnetic charges (monopoles).

Q.181 [Ans: (B)]: Taking the divergence of ∇ × H = J + ∂D/∂t gives 0 = ∇ ⋅ J + ∂(∇ ⋅ D)/∂t = ∇ ⋅ J + ∂ρv/∂t, satisfying the charge continuity equation. Q.182 [Ans: (A)]: vp = c / √(μrεr) = 3 × 108 / √4 = 1.5 × 108 m/s; η = η0 √(μr/εr) = 120π / √4 = 60π ≈ 188.5 Ω. Q.183 [Ans: (A)]: Skin depth δ = 1 / √(π f μ σ) ∝ 1 / √f. Increasing frequency by a factor of 100 reduces δ by √100 = 10, giving δ2 = 0.01 mm. Q.184 [Ans: (A)]: Argument (ωt − βz) indicates propagation in +az; time-average power density Pavg = (¦E0¦2 / (2η0)) az = (120π)2 / (2 × 120π) az = 60π az W/m2. Q.185 [Ans: (B)]: Equal orthogonal amplitudes (¦Ex¦ = ¦Ey¦ = E0) with a ±90° phase difference produce Circular Polarization.

Q.186 [Ans: (A)]: Γ = (η2 − η1) / (η2 + η1) = (40π − 120π) / (40π + 120π) = −80/160 = −0.5; τ = 1 + Γ = 2η2/(η1+η2) = +0.5. Q.187 [Ans: (A)]: For non-magnetic media (μ1 = μ2), the Brewster angle exists only for parallel (TM) polarization (Γ∥ = 0), leaving the reflected wave purely perpendicular (TE) polarized. Q.188 [Ans: (A)]: Γ = (ZL − Z0)/(ZL + Z0) = (150 − 50)/(150 + 50) = 100/200 = 0.5; VSWR = (1 + ¦Γ¦)/(1 − ¦Γ¦) = 1.5 / 0.5 = 3 (or directly ZL/Z0 = 150/50 = 3). Q.189 [Ans: (A)]: A quarter-wave transformer inverts impedance: Zin = Z02/ZL. Thus a shorted λ/4 stub looks like an open circuit (Zin = ∞) at its input. Q.190 [Ans: (A)]: Quarter-wave matching transformer impedance: Z0T = √(Z0 RL) = √(50 × 200) = √10000 = 100 Ω.

Q.191 [Ans: (A)]: Γ = (z − 1)/(z + 1): z = 1 ⇒ Γ = 0 (center); z = 0 ⇒ Γ = −1 (leftmost); z = ∞ ⇒ Γ = +1 (rightmost); outer unit circle ¦Γ¦ = 1 represents pure reactances. Q.192 [Ans: (A)]: Heaviside's distortionless condition is R/L = G/C, which gives constant attenuation α = √(RG), constant impedance Z0 = √(L/C), and constant phase velocity vp = 1/√(LC). Q.193 [Ans: (A)]: For a > b, dominant mode is TE10 with cutoff wavelength λc10 = 2a = 6 cm, so fc10 = c / (2a) = 3 × 108 / 0.06 = 5 GHz. Q.194 [Ans: (A)]: Since Ez(x, y) = E0 sin(mπx/a) sin(nπy/b) vanishes identically if m = 0 or n = 0, both m ≥ 1 and n ≥ 1 are required, making TM11 the lowest-order TM mode. Q.195 [Ans: (A)]: √(1 − (fc/f)2) = √(1 − 0.36) = 0.8. Group velocity vg = c√(1 − (fc/f)2) = 0.8c, and phase velocity vp = c / √(1 − (fc/f)2) = 1.25c (with vp vg = c2).

Q.196 [Ans: (A)]: Universal aperture relation: D = 4π Ae / λ2. Isotropic D = 1 (0 dBi); Hertzian dipole D = 1.5 (1.76 dBi); λ/2 dipole D = 1.64 (2.15 dBi) with radiation resistance ~73 Ω. Q.197 [Ans: (B)]: By Friis formula, Pr / Pt = Gt Gr (λ / (4πR))2 ∝ 1 / (f2 R2). Doubling both R and f reduces Pr by a factor of 22 × 22 = 16 (i.e., 12 dB). Q.198 [Ans: (A)]: When elements spaced by λ/2 are fed in-phase (α = 0°), waves add constructively perpendicular to the axis (Broadside) and cancel along the axis (where path difference is λ/2 = 180°). Q.199 [Ans: (A)]: Numerical Aperture NA = √(n12 − n22) = √(2.25 − 2.1609) = √0.0891 ≈ 0.298 (and acceptance angle θa = sin−1(NA)). Q.200 [Ans: (B)]: For a monostatic radar, round-trip path changes at rate 2v, so Doppler shift fd = 2v / λ = (2 × 30 m/s) / (0.03 m) = 2000 Hz = 2 kHz.

SECTION X: COMPLETE MASTER ANSWER KEY (Q.1 – Q.200) Verified Answer Key for all 200 GATE Electronics One-Liner Questions organized by Subject Section

| Col 1 | Col 2 | Col 3 | Col 4 | Col 5 | Col 6 | Col 7 | Col 8 | Col 9 | Col 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q.1: (B) | Q.2: (B) | Q.3: (B) | Q.4: (B) | Q.5: (B) | Q.6: (A) | Q.7: (A) | Q.8: (B) | Q.9: (C) | Q.10: (B) |
| Q.11: (B) | Q.12: (B) | Q.13: (B) | Q.14: (B) | Q.15: (B) | Q.16: (B) | Q.17: (B) | Q.18: (B) | Q.19: (A) | Q.20: (D) |
| Q.21: (B) | Q.22: (B) | Q.23: (C) | Q.24: (B) | Q.25: (B) | Q.26: (A) | Q.27: (C) | Q.28: (B) | Q.29: (B) | Q.30: (C) |
| Q.31: (A) | Q.32: (B) | Q.33: (C) | Q.34: (B) | Q.35: (D) | Q.36: (C) | Q.37: (C) | Q.38: (B) | Q.39: (A) | Q.40: (A) |
| Q.41: (C) | Q.42: (A) | Q.43: (B) | Q.44: (B) | Q.45: (A) | Q.46: (A) | Q.47: (A) | Q.48: (B) | Q.49: (B) | Q.50: (B) |
| Q.51: (A) | Q.52: (C) | Q.53: (B) | Q.54: (B) | Q.55: (C) | Q.56: (A) | Q.57: (C) | Q.58: (A) | Q.59: (A) | Q.60: (B) |
| Q.61: (B) | Q.62: (A) | Q.63: (B) | Q.64: (B) | Q.65: (B) | Q.66: (A) | Q.67: (B) | Q.68: (A) | Q.69: (A) | Q.70: (A) |
| Q.71: (A) | Q.72: (B) | Q.73: (A) | Q.74: (A) | Q.75: (A) | Q.76: (B) | Q.77: (A) | Q.78: (B) | Q.79: (B) | Q.80: (B) |
| Q.81: (A) | Q.82: (B) | Q.83: (B) | Q.84: (A) | Q.85: (B) | Q.86: (B) | Q.87: (B) | Q.88: (B) | Q.89: (B) | Q.90: (A) |
| Q.91: (B) | Q.92: (B) | Q.93: (A) | Q.94: (C) | Q.95: (B) | Q.96: (B) | Q.97: (B) | Q.98: (B) | Q.99: (B) | Q.100: (B) |
| Q.101: (B) | Q.102: (A) | Q.103: (A) | Q.104: (A) | Q.105: (B) | Q.106: (A) | Q.107: (B) | Q.108: (C) | Q.109: (B) | Q.110: (B) |
| Q.111: (A) | Q.112: (B) | Q.113: (C) | Q.114: (C) | Q.115: (B) | Q.116: (B) | Q.117: (B) | Q.118: (B) | Q.119: (B) | Q.120: (B) |
| Q.121: (B) | Q.122: (A) | Q.123: (B) | Q.124: (B) | Q.125: (B) | Q.126: (A) | Q.127: (A) | Q.128: (B) | Q.129: (A) | Q.130: (B) |
| Q.131: (A) | Q.132: (C) | Q.133: (B) | Q.134: (A) | Q.135: (B) | Q.136: (B) | Q.137: (A) | Q.138: (A) | Q.139: (A) | Q.140: (A) |
| Q.141: (A) | Q.142: (A) | Q.143: (A) | Q.144: (B) | Q.145: (A) | Q.146: (A) | Q.147: (A) | Q.148: (A) | Q.149: (A) | Q.150: (A) |
| Q.151: (A) | Q.152: (A) | Q.153: (A) | Q.154: (B) | Q.155: (A) | Q.156: (A) | Q.157: (B) | Q.158: (A) | Q.159: (B) | Q.160: (B) |
| Q.161: (A) | Q.162: (A) | Q.163: (A) | Q.164: (B) | Q.165: (A) | Q.166: (A) | Q.167: (A) | Q.168: (A) | Q.169: (B) | Q.170: (A) |
| Q.171: (A) | Q.172: (A) | Q.173: (A) | Q.174: (B) | Q.175: (A) | Q.176: (A) | Q.177: (A) | Q.178: (A) | Q.179: (A) | Q.180: (B) |
| Q.181: (B) | Q.182: (A) | Q.183: (A) | Q.184: (A) | Q.185: (B) | Q.186: (A) | Q.187: (A) | Q.188: (A) | Q.189: (A) | Q.190: (A) |
| Q.191: (A) | Q.192: (A) | Q.193: (A) | Q.194: (A) | Q.195: (A) | Q.196: (A) | Q.197: (B) | Q.198: (A) | Q.199: (A) | Q.200: (B) |